O-RAN.WG11.TR.PQC-Security.0-R005-v02.00

Technical Report

O-RAN Work Group 11 (Security Work Group) Study on Post-Quantum Cryptography

Copyright © 2026 by the O-RAN ALLIANCE e.V. The copying or incorporation into any other work of part or all of the material available in this document in any form without the prior written permission of O-RAN ALLIANCE e.V. is prohibited, save that you may print or download extracts of the material of this document for your personal use, or copy the material of this document for the purpose of sending to individual third parties for their information provided that you acknowledge O-RAN ALLIANCE as the source of the material and that you inform the third party that these conditions apply to them and that they must comply with them. O-RAN ALLIANCE e.V., Buschkauler Weg 27, 53347 Alfter, Germany Register of Associations, Bonn VR 11238, VAT ID DE321720189

- Contents

List of tables	3

Foreword	4

Modal verbs terminology	4

Introduction	4

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definition of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Post-Quantum Cryptography background	10

5	Cryptographic inventory	10

Annex: Change history/Change request (history)	16

# List of tables

Table 5-1: Cryptographic inventory	10

# Foreword

This Technical Report (TR) has been produced by WG11 of the O-RAN ALLIANCE.

The content of the present document is subject to continuing work within O-RAN and may change following formal O-RAN approval. Should the O-RAN ALLIANCE modify the contents of the present document, it will be re-released by O-RAN with an identifying change of version date and an increase in version number as follows:

version xx.yy.zz

where:

xx:	the first digit-group is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc. (the initial approved document will have xx=01).  Always 2 digits with leading zero if needed.

yy:	the second digit-group is incremented when editorial only changes have been incorporated in the document. Always 2 digits with leading zero if needed.

zz:	the third digit-group included only in working versions of the document indicating incremental changes during the editing process. External versions never include the third digit-group.  Always 2 digits with leading zero if needed.

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the O-RAN Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in O-RAN deliverables except when used in direct citation.

# Introduction

The increasing sophistication of quantum computing poses a significant threat to the security of classical asymmetric cryptographic algorithms currently used in O-RAN networks. As quantum computers become more powerful, they will be capable of breaking asymmetric classical encryption algorithms, potentially compromising the confidentiality, integrity, and authenticity of sensitive information exchanged between network entities. This vulnerability has far-reaching implications for the security and trustworthiness of O-RAN networks, which are designed to provide secure and reliable communication services for critical infrastructure and applications.

The migration to Quantum Safe Cryptography (QSC) is essential to ensure the long-term security and resilience of O-RAN networks against potential quantum computer-based attacks. QSC refers to cryptographic techniques and algorithms that are resistant to attacks by both classical and quantum computers. By migrating to QSC, O-RAN networks can maintain their security posture and prevent potential disruptions to services and operations.

This technical report contains a cryptographic inventory, covering all cryptographic algorithms employed in O-RAN specification at this time.

# Scope

The present document presents the Post-Quantum Cryptography study as it applies to O-RAN architectural elements.

# References

## Normative references

Not applicable.

## Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies. In the case of a reference to a 3GPP document, a non-specific reference implicitly refers to the latest version of that document in Release 18, or the latest 3GPP release prior to Release 18 that includes that document.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, O-RAN cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document, but they assist the user with regard to a particular subject area.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications ".

- NIST Post-Quantum Cryptography, call for papers; https://csrc.nist.gov/Projects/post-quantum-cryptography/post-quantum-cryptography-standardization/Call-for-Proposals.

- FIPS 203: Module-Lattice-Based Key-Encapsulation Mechanism Standard; https://csrc.nist.gov/pubs/fips/203/final.

- FIPS 204: Module-Lattice-Based Digital Signature Standard; https://csrc.nist.gov/pubs/fips/204/final.

- FIPS 205: Stateless Hash-Based Digital Signature Standard; https://csrc.nist.gov/pubs/fips/205/final.

- NCSC whitepaper: Next steps in preparing for post-quantum cryptography; https://www.ncsc.gov.uk/whitepaper/next-steps-preparing-for-post-quantum-cryptography.

- ETSI TR 103 619 V1.1.1 (2020-07), " Migration strategies and recommendations to Quantum Safe schemes".

- ETSI TC CYBER, ETSI QSC, " Quantum Safe Cryptographic Protocol Inventory", CYBERQSC(23)032007r2_QSC_Protocol_Inventory_LSOut.pdf.

- GSMA, "LS regarding the publication of the Post Quantum Cryptography – Guidelines for Telecom Use Cases document in Feb 24".

- GSMA, "Post Quantum Cryptography – Guidelines for Telecom Use Cases"; https://www.gsma.com/newsroom/wp-content/uploads/PQ.03-Post-Quantum-Cryptography-Guidelines-for-Telecom-Use-v1.0.pdf.

- NIST, "Post-Quantum Cryptography"; https://www.nist.gov/programs-projects/post-quantum-cryptography.

- O-RAN.WG4.TS.MP: "O-RAN WG4 Management Plane Specification".

- O-RAN.WG10.TS.O1-Interface: "O-RAN WG10 O-RAN O1 Interface Specification".

- O-RAN.WG11.TS.SecProtSpec: "O-RAN WG11 Security Protocols Specifications".

- IETF RFC 5656: "Elliptic Curve Algorithm Integration in the Secure Shell Transport Layer".

- IETF RFC 8709: "Ed25519 and Ed448 Public Key Algorithms for the Secure Shell (SSH) Protocol".

- IETF RFC 5647: "AES Galois Counter Mode for the Secure Shell Transport Layer Protocol".

- IETF RFC 4344: "The Secure Shell (SSH) Transport Layer Encryption Modes".

- IETF RFC 4419: "Diffie-Hellman Group Exchange for the Secure Shell (SSH) Transport Layer Protocol".

- IETF RFC 8270. "Increase the Secure Shell Minimum Recommended Diffie-Hellman Modulus Size to 2048 Bits".

- IETF RFC 8731: "Secure Shell (SSH) Key Exchange Method Using Curve25519 and Curve448".

- IETF RFC 6668: "SHA-2 Data Integrity Verification for the Secure Shell (SSH) Transport Layer Protocol".

- IETF RFC 4418: "UMAC: Message Authentication Code using Universal Hashing”.

- O-RAN.WG2.A1TP: "A1 interface: Transport Protocol".

- O-RAN.WG2.R1TP: "Transport Protocols for R1 Services".

- O-RAN.WG3.Y1GAP: "Y1 interface: General Aspects and Principles".

- IETF RFC 5116: "An Interface and Algorithms for Authenticated Encryption".

- IETF RFC 8422: "Elliptic Curve Cryptography (ECC) Cipher Suites for Transport Layer Security (TLS) Versions 1.2 and Earlier".

- IETF RFC 7919: "Negotiated Finite Field Diffie-Hellman Ephemeral Parameters for Transport Layer Security (TLS) ".

- IETF RFC 6234: "US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF) ".

- IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

- IETF RFC 8439: "ChaCha20 and Poly1305 for IETF Protocols".

- BSI TR-02102-2: "Cryptographic Mechanisms: Recommendations and Key Lengths: Use of Transport Layer Security (TLS) ".

- NIST SP 800-38D: "Recommendation for Block Cipher Modes of Operation: Galois/Counter Mode (GCM) and GMAC".

- IETF RFC 8032: "Edwards-Curve Digital Signature Algorithm (EdDSA)".

- IETF RFC 7518: "JSON Web Algorithms (JWA)".

- O-RAN.WG3.RICARCH: "O-RAN Near-RT RIC Architecture".

- O-RAN.WG6.AAL-GAnP: "O-RAN Acceleration Abstraction Layer General Aspects and Principles".

- O-RAN.WG6.O2-GA&P: “O-RAN WG6 O2 Interface General Aspects and Principles”.

- IETF RFC	 7516: "JSON Web Encryption (JWE)".

- IEEE Std 802.1AE-2018, "IEEE Standard for Local and metropolitan area networks — Media Access Control (MAC) Security", IEEE, 2018.

- IEEE 802.1X-2020, "IEEE Standard for Local and Metropolitan Area Networks--Port-Based Network Access Control", IEEE, 2020-02-28.

- O-RAN.WG4.TS.CUS: "O-RAN WG11 Study on Security for Fronthaul CUS-Plane".

- IETF RFC 4493: "The AES-CMAC Algorithm".

- IETF RFC 5216: "The EAP-TLS Authentication Protocol".

- IETF RFC 9190: "EAP-TLS 1.3: Using the Extensible Authentication Protocol with TLS 1.3".

- IEEE Std 1588 – 2019, "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", IEEE, 2019.

- IETF RFC 2410: "The NULL Encryption Algorithm and Its Use with IPsec".

- IETF RFC 2404: "The Use of HMAC-SHA-1-96 within ESP and AH".

- IETF RFC 3526: "More Modular Exponential (MODP) Diffie-Hellman groups for Internet Key Exchange (IKE)".

- IETF RFC 3566: "The AES-XCBC-MAC-96 Algorithm and Its Use with IPsec".

- IETF RFC 3602: "The AES-CBC Cipher Algorithm and Its Use with IPsec".

- IETF RFC 4106: "The Use of Galois/Counter Mode (GCM) in IPsec Encapsulating Security Payload (ESP)".

- IETF RFC 4309: "Using Advanced Encryption Standard (AES) CCM Mode with IPsec Encapsulating Security Payload (ESP)".

- IETF RFC 4543: "The Use of Galois Message Authentication Code (GMAC) in IPsec ESP and AH".

- IETF RFC 4868: "Using HMAC-SHA-256, HMAC-SHA-384, and HMAC-SHA-512 with IPsec".

- IETF RFC 5114: "Additional Diffie-Hellman Groups for Use with IETF Standards".

- IETF RFC 5282: "Using Authenticated Encryption Algorithms with the Encrypted Payload of the Internet Key Exchange version 2 (IKEv2) Protocol".

- IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

- IETF RFC 7634: "ChaCha20, Poly1305, and Their Use in the Internet Key Exchange Protocol (IKE) and IPsec".

- IETF RFC 8221: "Cryptographic Algorithm Implementation Requirements and Usage Guidance for Encapsulating Security Payload (ESP) and Authentication Header (AH)".

- IETF RFC 8247: "Algorithm Implementation Requirements and Usage Guidance for the Internet Key Exchange Protocol Version 2 (IKEv2) ".

- O-RAN.WG3.TS.E2GAP: "O-RAN WG3 E2 General Aspects and Principles (E2GAP)".

- O-RAN.WG1.TS.OAD: "O-RAN Architecture Description".

- IETF RFC 7519: "JSON Web Token (JWT)".

- IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

- IETF RFC 2631: "Diffie-Hellman Key Agreement Method".

- IETF RFC 3370: "Cryptographic Message Syntax (CMS) Algorithms".

- IETF RFC 3394: "Advanced Encryption Standard (AES) Key Wrap Algorithm".

- IETF RFC 4210: "Internet X.509 Public Key Infrastructure Certificate Management Protocol (CMP)".

- IETF RFC 4211: "Internet X.509 Public Key Infrastructure Certificate Request Message Format (CRMF)".

- IETF RFC 5753: "Use of Elliptic Curve Cryptography (ECC) Algorithms in Cryptographic Message Syntax (CMS)".

- IETF RFC 8017: "PKCS #1: RSA Cryptography Specifications Version 2.2".

- IETF RFC 8018: "PKCS #5: Password-Based Cryptography Specification Version 2.1".

- IETF RFC 8702: "Use of the SHAKE One-Way Hash Functions in the Cryptographic Message Syntax (CMS)".

- IETF RFC 9045: "Algorithm Requirements Update to the Internet X.509 Public Key Infrastructure Certificate Request Message Format (CRMF)".

- IETF RFC 9481: "Certificate Management Protocol (CMP) Algorithms".

- FIPS SP 800-185: SHA-3 Derived Functions: cSHAKE, KMAC, TupleHash, and ParallelHash; https://csrc.nist.gov/publications/detail/sp/800-185/final.

- FIPS 180-4: Secure Hash Standard (SHS); https://csrc.nist.gov/pubs/fips/180-4/upd1/final.

- FIPS 186-5: Digital Signature Standard (DSS); https://csrc.nist.gov/pubs/fips/186-5/final.

- FIPS 197: Advanced Encryption Standard (AES); https://csrc.nist.gov/pubs/fips/197/final.

- FIPS 202: SHA-3 Standard: Permutation-Based Hash and Extendable-Output Functions; https://csrc.nist.gov/pubs/fips/202/final.

- NIST SP 800-186: "Recommendations for Discrete Logarithm-based Cryptography: Elliptic Curve Domain Parameters".

- NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197)".

- NIST SP 800-38A: "Recommendation for Block Cipher Modes of Operation: Methods and Techniques".

- IETF RFC 8017: "PKCS #1: RSA Cryptography Specifications Version 2.2".

- Void.

- NIST: "SHA-3 Standard: Permutation-Based Hash and Extendable-Output Function (FIPS PUB 202)".

- NIST SP 800-90A: "Recommendation for Random Number Generation Using Deterministic Random Bit Generators".

- NIST SP 800-90B: "Recommendation for the Entropy Sources Used for Random Bit Generation".

- NIST SP 800-90C: "Recommendation for Random Bit Generator (RBG) Constructions".

- BSI: "Documentation and Analysis of the Linux Random Number Generator".

# Definition of terms, symbols and abbreviations

## Terms

For the purposes of the present document, the terms given in [i.1] and the following apply:

Post-Quantum Cryptography: An area of cryptography that researches and advances the use of quantum-resistant primitives, with the goal of keeping existing public key infrastructure intact in a future era of quantum computing. Intended to be secure against both quantum and classical computers and deployable without drastic changes to existing communication protocols and networks. [i.11]

Quantum Safe Cryptography: see Post-Quantum Cryptography

## Symbols

Void.

## Abbreviations

For the purposes of the present document, the abbreviations [given in [i.1] and the following] apply:

AES	Advanced Encryption Standard

CBC	Cipher Block Chaining

CBOM	Cryptographic bill of material

CMAC	Cipher-based Message Authentication Code

CMP	Certificate Management Protocol

CUS-Plane	Control, User and Synchronization Plane

DH	Diffie-Hellman (Key Exchange)

DTLS	Datagram Transport Layer Security

ECDSA	Elliptic Curve Digital Signature Algorithm

ETM	Encrypt-then-MAC

FH	Fronthaul

GCM	Galois/Counter Mode

HMAC	Hash-based message authentication code

JSON	JavaScript Object Notation

JWA	JSON Web Algorithms

JWE	JSON Web Encryption

JWS	JSON Web Signature

JWT	JSON Web Token

MACsec	Media Access Control security

MGF	Mask generation function

MKA	MACsec Key Agreement

OFH	Open Fronthaul

PKC	Public-Key Cryptography

PKCS	Public-Key Cryptography Standards

PQC	Post-Quantum Cryptography

PTP	Precision Time Protocol

PSS	Probabilistic Signature Scheme

QSC	Quantum Safe Cryptography

RSA	Rivest–Shamir–Adleman

RSAASSA	RSA Probabilistic Signature Scheme with Appendix (often referred to as RSASSA)

SHA	Secure Hash Algorithm

SSH	Secure Shell

TLS	Transport Layer Security

XCBC	eXtended Cipher Block Chaining

XPN	eXtended Packet Number

# Post-Quantum Cryptography background

NIST has initiated the process of the Post Quantum Cryptography (PQC) standardization in 2017 [i.2]. On 5th Jul 2022, they announced the standard candidates for public-key encryption:

- CRYSTALS-Kyber,

- for digital signature: CRYSTALS-Dilithium, FALCON and SPHINCS+.

On 13th Aug 2024, NIST published FIPS 203 [i.3], FIPS 204 [i.4] and FIPS 205 [i.5], specifying algorithms derived from CRYSTALS-Dilithium, CRYSTALS-Kyber  and SPHINCS+: ML-KEM, ML-DSA and SLH-DSA. A future FIPS 206 is planned to specify FN-DSA, based on FALCON.

Further input from NIST:

“Quantum computers would completely break many public-key cryptosystems, including RSA, DSA, and elliptic curve cryptosystems. These cryptosystems are used to implement digital signatures and key establishment and play a crucial role in ensuring the confidentiality and authenticity of communications on the Internet and other networks.” [i.2]

Further input from NCSC:

“In contrast with PKC, the security of symmetric cryptography is not significantly impacted by quantum computers, and existing symmetric algorithms with at least 128-bit keys (such as AES) can continue to be used. The security of hash functions such as SHA-256 is also not significantly affected, and secure hash functions can also continue to be used.” [i.6]

The ETSI QSC group has developed a document outlining a Quantum Safe Cryptography migration strategy and recommendations for transitioning to quantum-safe schemes [i.7]. A key component of this strategy is the creation of a cryptographic inventory or cryptographic bill of material (CBOM) to support the development of a preparation and PQC migration plan, as well as the execution of the migration.

According to the LS [i.8], the ETSI QSC is seeking additional information, such as a "detailed list of cryptographic algorithms and/or protocol implementations." This list may include "guidance on algorithm exposure, effective key lengths, potential for hybrid deployment, and potential for crypto agility."

In a related development, as noted in another LS [i.9], the GSMA has produced a Guidelines for Telecom Use Cases document [i.10], which includes the development of a "comprehensive cryptographic inventory" to some extent.

# Cryptographic inventory

Table 5-1: Cryptographic inventory

| Security Protocol | Traditional Algorithm | Algorithm Development Organisation and document(s) | Algorithm used for | O-RAN Adoption (EXAMPLE: Interfaces, APIs) | O-RAN specifi-cation(s) |
| --- | --- | --- | --- | --- | --- |
| TLS 1.2, DTLS 1.2 | AES-128-GCM | RFC 5116 [i.27] (NOTE 3) | Cipher suites | OFH m-plane [i.12] O1 [i.13] A1 [i.24] R1 [i.25] Y1 [i.26] O2 [i.39] SMO External Interface [i.64] | SPS [i.14] |
|  | AES-256-GCM |  |  |  |  |
|  | secp256r1 | RFC 8422 [i.28] | Key exchange |  |  |
|  | secp384r1 |  |  |  |  |
|  | ffdhe2048 | RFC 7919 [i.29] |  |  |  |
|  | ffdhe3072 |  |  |  |  |
|  | ffdhe4096 |  |  |  |  |
|  | ffdhe6144 |  |  |  |  |
|  | ffdhe8192 |  |  |  |  |
|  | SHA-256 | RFC 6234 [i.30] | Hash functions |  |  |
|  | SHA-384 |  |  |  |  |
|  | ecdsa | RFC 8422 [i.28] | Signature algorithms |  |  |
|  | rsa_pss_rsae_* | RFC 8446 [i.31] |  |  |  |
|  | rsa_pkcs1 | RFC 8032 [i.35] (NOTE 2) |  |  |  |
| TLS 1.3, DTLS 1.3 | AES-128-GCM | RFC 5116 [i.27] (NOTE 3) | Cipher suites |  |  |
|  | AES-256-GCM |  |  |  |  |
|  | ChaCha20-Poly1305 | RFC 8439 [i.32] |  |  |  |
|  | secp256r1 | RFC 8422 [i.28] | Key exchange |  |  |
|  | secp384r1 |  |  |  |  |
|  | secp521r1 |  |  |  |  |
|  | SHA-256 | RFC 6234 [i.30] | Hash functions |  |  |
|  | SHA-384 |  |  |  |  |
|  | SHA-512 |  |  |  |  |
|  | ecdsa_secp256r1_sha256 | RFC 8446 [i.31] | Signature algorithms |  |  |
|  | ecdsa_secp384r1_sha384 |  |  |  |  |
|  | ecdsa_secp521r1_sha512 |  |  |  |  |
|  | rsa_pss_rsae_sha256 |  |  |  |  |
|  | rsa_pss_rsae_sha384 |  |  |  |  |
|  | rsa_pss_rsae_sha512 |  |  |  |  |
|  | rsa_pss_pss_sha256 |  |  |  |  |
|  | rsa_pss_pss_sha384 |  |  |  |  |
|  | rsa_pss_pss_sha512 |  |  |  |  |
|  | ed25519 | RFC 8032 [i.35] |  |  |  |
| SSH | ecdsa-sha2-nistp256 | RFC 5656 [i.15][i.15] | Key agreement | OFH m-plane [i.12] O1 [i.13] SMO External Interface [i.64] | SPS [i.14] |
|  | ecdsa-sha2-nistp384 |  |  |  |  |
|  | ecdsa-sha2-nistp521 |  |  |  |  |
|  | ssh-ed25519 | RFC 8709 [i.16] |  |  |  |
|  | ssh-ed448 |  |  |  |  |
|  | aes256-gcm | RFC 5647 [i.17] | Symmetric encryption and decryption |  |  |
|  | aes128-gcm |  |  |  |  |
|  | aes256-ctr | RFC 4344 [i.18] |  |  |  |
|  | aes192-ctr |  |  |  |  |
|  | aes128-ctr |  |  |  |  |
|  | ecdh-sha2-nistp521 | RFC 5656 [i.15] | Key exchange |  |  |
|  | ecdh-sha2-nistp384 |  |  |  |  |
|  | ecdh-sha2-nistp256 |  |  |  |  |
|  | diffie-hellman-group-exchange-sha256 | RFC 4419 [i.19], updated by RFC 8270 [i.20] |  |  |  |
|  | curve25519-sha256 | RFC 8731 [i.21] |  |  |  |
|  | hmac-sha2-512-etm | RFC 6668 [i.22] (NOTE 1) | Message authentication codes |  |  |
|  | hmac-sha2-512 |  |  |  |  |
|  | hmac-sha2-256-etm |  |  |  |  |
|  | hmac-sha2-256 |  |  |  |  |
|  | umac-128 | RFC 4418 [i.23] |  |  |  |
| IPsec | ENCR_NULL | RFC 2410 [i.48] | ESP encryption algorithms (RFC 8221 – [i.61]) | E2 [i.63] | SPS [i.14] |
|  | ENCR_AES_CBC | RFC 3602 [i.52] |  |  |  |
|  | ENCR_AES_CCM_8 | RFC 4309 [i.54] |  |  |  |
|  | ENCR_AES_GCM_16 | RFC 4106 [i.53] |  |  |  |
|  | ENCR_CHACHA20_POLY1305 | RFC 7634 [i.60] |  |  |  |
|  | AUTH_NONE (NOTE 7) | RFC 7296 [i.59], RFC 5282 [i.58] | ESP and AH Authentication Algorithms (RFC 8221 – [i.61]) |  |  |
|  | AUTH_HMAC_SHA1_96 | RFC 2404 [i.49], RFC 7296 [i.59] |  |  |  |
|  | AUTH_AES_XCBC_96 (NOTE 8) | RFC 3566 [i.51], RFC 7296 [i.59] |  |  |  |
|  | AUTH_AES_128_GMAC | RFC 4543 [i.55] |  |  |  |
|  | AUTH_AES_256_GMAC | RFC 4543 [i.55] |  |  |  |
|  | AUTH_HMAC_SHA2_256_128 | RFC 4868 [i.56] |  |  |  |
|  | AUTH_HMAC_SHA2_512_256 | RFC 4868 [i.56] |  |  |  |
|  | ENCR_AES_CBC | RFC 3602 [i.52] | IKEv2 Encryption Algorithm Transforms (RFC 8247 – [i.62]) |  |  |
|  | ENCR_CHACHA20_POLY1305 | RFC 7634 [i.60] |  |  |  |
|  | ENCR_ASES_GCM_16 | RFC 4106 [i.53] |  |  |  |
|  | ENCR_AES_CCM_8 | RFC 4309 [i.54] |  |  |  |
|  | AUTH_HMAC_SHA2_256_128 | RFC 4868 [i.56] | IKEv2 Integrity Algorithm Transforms (RFC 8247 – [i.62]) |  |  |
|  | AUTH_HMAC_SHA2_512_256 | RFC 4868 [i.56] |  |  |  |
|  | AUTH_HMAC_SHA1_96 | RFC 2404 [i.49], RFC 7296 [i.59] |  |  |  |
|  | AUTH_AES_XCBC_96 | RFC 3566 [i.51], RFC 7296 [i.59] |  |  |  |
|  | PRF_HMAC_SHA2_256 | RFC 4868 [i.56] | IKEv2 Pseudorandom Function Transforms (RFC 8247 – [i.62]) |  |  |
|  | PRF_HMAC_SHA2_512 | RFC 4868 [i.56] |  |  |  |
|  | PRF_HMAC_SHA1 | RFC 2404 [i.49] |  |  |  |
|  | PRF_AES128_XCBC | RFC 4344 [i.18] |  |  |  |
|  | DH 14 – 2048-bit MODP Group | RFC 3526 [i.50] | IKEv2 Diffie-Hellman Group Transforms (RFC 8247 – [i.62]) |  |  |
|  | DH 19 – 256-bit random ECP group | RFC 5114 [i.57] |  |  |  |
| JWS (NOTE 5) | HS256 (HMAC using SHA-256) | RFC 7518 [i.36] | Message authentication codes | A1 [i.24] R1 [i.25] Y1 [i.26] Near-RT RIC APIs [i.37] AALI-C-Mgmt interface [i.38] O2 [i.39] | SPS [i.14] |
|  | HS384 (HMAC using SHA-384) |  |  |  |  |
|  | HS512 (HMAC using SHA-512) |  |  |  |  |
|  | RS256 (RSASSA-PKCS1-v1_5 using SHA-256) | RFC 7518 [i.36] | Signature algorithms |  |  |
|  | RS384 (RSASSA-PKCS1-v1_5 using SHA-384) |  |  |  |  |
|  | RS512 (RSASSA-PKCS1-v1_5 using SHA-512) |  |  |  |  |
|  | ES256 (ECDSA using P-256 curve and SHA-256) |  |  |  |  |
|  | ES384 (ECDSA using P-384 curve and SHA-384) |  |  |  |  |
|  | ES512 (ECDSA using P-512 curve and SHA-512) |  |  |  |  |
|  | PS256 (RSASSA-PSS using SHA-256 and MGF1 with SHA-256) |  |  |  |  |
|  | PS384 (RSASSA-PSS using SHA-384 and MGF1 with SHA-384) |  |  |  |  |
|  | PS512 (RSASSA-PSS using SHA-512 and MGF1 with SHA-512) |  |  |  |  |
| JWE | A128CBC-HS256 (AES_128_CBC_HMAC_SHA_256) | RFC 7518 [i.36] RFC 7516 [i.40], clause 4.1.2. | Encryption algorithms | N/A | Not yet specified |
|  | A192CBC-HS384 (AES_192_CBC_HMAC_SHA_384) |  |  |  |  |
|  | A256CBC-HS512 (AES_256_CBC_HMAC_SHA_512) |  |  |  |  |
|  | A128GCM (AES GCM using 128-bit key) |  |  |  |  |
|  | A192GCM (AES GCM using 192-bit key) |  |  |  |  |
|  | A256GCM (AES GCM using 256-bit key) |  |  |  |  |
| OAuth 2.0 | (NOTE 6) | RFC 6749 [i.66] | Authorization framework | A1 [i.24], O2 [i.39], R1 [i.25], Y1 [i.26], Near RT RIC APIs [i.37] | SPS [i.14] |
| MACsec (NOTE 4) | GCM-AES-128 | IEEE 802.1AE-2018 [i.41] IEEE 802.1X-2020 [i.42] | Symmetric encryption and decryption, Message authentication codes | OFH CUS Plane Security [i.43] OFH m-plane [i.12] | SPS [i.14] OFH m-plane [i.12] |
|  | GCM-AES-256 |  |  |  |  |
|  | GCM-AES-XPN-128 |  |  |  |  |
|  | GCM-AES-XPN-256 |  |  |  |  |
|  | AES-CMAC | RFC 4493 [i.44] | Key derivation |  |  |
| PTP | HS256 (HMAC using SHA-256) | IEEE 1588 [i.47], clause 16.14 | Message authentication codes | OFH CUS Plane Security [i.43] OFH m-plane [i.12] | SPS [i.14] OFH m-plane [i.12] |
|  | HS384 (HMAC using SHA-384) |  |  |  |  |
|  | HS512 (HMAC using SHA-512) |  |  |  |  |
| CMP | SHA2: sha224 sha256 sha384 shat512 | RFC 9481 [i.77] Sec 2.1, FIPS Pub 180-4 [i.79] | Message Digest Algorithms | OFH m-plane [i.12] | SPS [i.14] |
|  | SHAKE: shake128 shake256 | RFC 9481 [i.77] Sec 2.2, FIPS Pub 202 [i.82] |  |  |  |
|  | RSA: RSASSA-PSS RSASSA-PSS-SHAKE128 RSASSA-PSS-SHAKE256 sha224WithRSAEncryption sha256WithRSAEncryption sha384WithRSAEncryption sha512WithRSAEncryption | RFC 9481 [i.77] Sec 3.1, RFC 8017 [i.73] | Signature Algorithms |  |  |
|  | ECDSA: ecdsa-with-SHA224 ecdsa-with-SHA256 ecdsa-with-SHA384 ecdsa-with-SHA512 ecdsa-with-shake128 ecdsa-with-shake256 | RFC 9481 [i.77] Sec 3.2, FIPS Pub 186-5 [i.80] |  |  |  |
|  | EdDSA: Ed25519 Ed448 | RFC 9481 [i.77] Sec 3.2, FIPS Pub 186-5 [i.80] |  |  |  |
|  | Diffie-Hellman: ESDH | RFC 9481 [i.77] Sec 4.1.1 RFC 2631 [i.67] RFC 3370 [i.68] | Key Management Algorithms (KMA) – Key Agreement Algorithms |  |  |
|  | ECDH: dhSinglePass-stdDH-sha224kdf dhSinglePass-stdDH-sha256kdf dhSinglePass-stdDH-sha384kdf dhSinglePass-stdDH-sha512kdf dhSinglePass-cofactorDH-sha224kdf dhSinglePass-cofactorDH-sha256kdf dhSinglePass-cofactorDH-sha384kdf dhSinglePass-cofactorDH-sha512kdf mqvSinglePass-sha224kdf mqvSinglePass-sha256kdf mqvSinglePass-sha384kdf mqvSinglePass-sha512kdf X25519 X448 | RFC 9481 [i.77] Sec 4.1.2 RFC 5753 [i.72] |  |  |  |
|  | RSA: RSA (PKCS #1 v1.5) RSAES-OAEP | RFC 9481 [i.77] Sec 4.2 | KMA – Key Transport Algorithms |  |  |
|  | AES Key Wrap: aes128-wrap aes192-wrap aes256-wrap | RFC 9481 [i.77] Sec 4.3, FIPS Pub 197 [i.81], RFC 3394 [i.69] | KMA – Symmetric Key-Encryption Algorithms |  |  |
|  | PBKDF2: PBKDF2 | RFC 9481 [i.77] Sec 4.4, RFC 8018 [i.74] | KMA – Key Derivation Algorithms |  |  |
|  | AES-CBC: aes128-CBC aes192-CBC aes256-CBC | RFC 9481 [i.77] Sec 5, FIPS Pub 197 [i.81] | Content-Encryption Algorithms |  |  |
|  | PasswordBased MAC: PasswordBasedMac | RFC 9481 [i.77] Sec 6,1.1 RFC 4210 [i.70] RFC 4211 [i.71] RFC 9045 [i.76] | Message Authentication Code (MAC) Algorithms – Password -Based MAC |  |  |
|  | PBMAC1: PBMAC1 | RFC 9481 [i.77] Sec 6,1.2 RFC 8018 [i.74] |  |  |  |
|  | SHA2-Based HMAC: hmacWithSHA224 hmacWithSHA256 hmacWithSHA384 hmacWithSHA512 | RFC 9481 [i.77] Sec 6,2.1 RFC 8018 [i.74] | MAC Algorithms – Symmetric Key-Based MAC |  |  |
|  | AES-GMAC: aes128-GMAC aes192-GMAC aes256-GMAC | RFC 9481 [i.77] Sec 6.2.2 FIPS Pub 197 [i.81] FIPS SP 800-38D [i.34] |  |  |  |
|  | SHAKE-Based KMAC: KMACWithSHAKE128 KMACWithSHAKE256 | RFC 9481 [i.77] Sec 6.2.3 RFC 8702 [i.75] FIPS SP 800-185 [i.78] |  |  |  |
| Crypto-graphic operations (see [i.14], section 5) | RSASSA-PSS RSA_PKCS1_V1_5 | FIPS Pub 186-5 [i.80] | Signature generation and verification | Application package authenticity and integrity | SPS [i.14] |
|  | ECDSA using NIST-approved curves (P-256, P-384, or P-521) | FIPS Pub 186-5 [i.80], SP 800-186 [i.83] |  |  |  |
|  | AES_128, AES_192 and AES_256 | FIPS 197 [i.84], SP 800-38A [i.85] (operation modes) | Symmetric Encryption/Decryption | Confidentiality of data |  |
|  | RSAES-OAEP | IETF RFC 8017 [i.86] | Asymmetric Encryption/Decryption |  |  |
|  | SHA-2 family (SHA- 224, SHA-256, SHA-384, SHA-512, SHA-512/224 and SHA-512/256) SHA-3 family (SHA3-224, SHA3-256, SHA3-384, and SHA3-512) | FIPS 180 [i.79] for SHA2 FIPS 202 [i.88] for SHA 3 | Hashing | Integrity of data |  |
|  | DRBGs (CTR_DRBG (AES), HMAC_DRBG (SHA-256), Hash_DRBG (SHA-256)) | NIST SP 800-90A [i.89] NIST SP 800-90B [i.90] NIST SP 800-90C [i.91] | Random Number Generation (RNG) | Cryptographic keys for encryption and digital signatures, random nonces for secure operations |  |
|  | System RNG EXAMPLE: Linux getrandom(), /dev/urandom | For Linux: BSI [i.92] |  | Unique instance IDs, secure entropy to seed DRBGs |  |
| NOTE 1: ETM (Encrypt-then-MAC) is not specified for SSH yet is a widely used and recommended practice within the SSHv2 protocol, and broadly supported in available implementations, due to its security- related advantages. NOTE 2: BSI recommends phasing out by 2025 [i.33]. NOTE 3: The related authenticated encryption algorithm is defined in [i.34] NOTE 4: EAP-TLS is based on TLS 1.2 (see IETF RFC 5216 [i.45]) or TLS 1.3 (see IETF RFC 9190 [i.46]) NOTE 5: JWT is based on JWS and JWE (see IETF RFC 7519 [i.65]) NOTE 6: OAuth 2.0 is based on TLS, JWT, and JWE. NOTE 7: AUTH_NONE MUST be supported as per IETF RFC 7296 [i.59] and IETF RFC 5282 [i.58] and MUST NOT be supported when ESP uses Authenticated Encryption with Associated Data (AEAD) cipher. NOTE 8: AUTH_AES_XCBC_96 SHOULD be supported as per IETF RFC 3566 [i.51] and IETF RFC 7296 [i.59], and MAY be support in case of Interoperability with IoT. |  |  |  |  |  |

NOTE 1:	RADIUS with EAP-TLS is not covered in Table 5-1 as the use of quantum-safe TLS for EAP-TLS is sufficient to render RADIUS over EAP-TLS quantum-safe without altering the RADIUS protocol itself.

NOTE 2:	LDAP with STARTTLS is not covered in Table 5-1 as the use of quantum-safe TLS for STARTTLS is sufficient to render LDAP over STARTTLS quantum-safe without altering the LDAP protocol itself.

# Annex:
Change history/Change request (history)

| Date | Revision | Description |
| --- | --- | --- |
| 2025.02.04 | 00.01 | First draft |
| 2025.02.25 | 00.02 | Merged NOK-0244, NOK-0245 |
| 2025.03.04 | 00.03 | Merged NOK-0256 |
| 2025.04.01 | 00.04 | Merged NOK-0268 |
| 2025.04.14 | 00.05 | Merged SYM-0084, NIST.AO-0012, NIST-0013 |
| 2025.05.27 | 00.06 | Merged NEC-0056 |
| 2025.06.10 | 00.07 | Merged NIST-0018, NIST-0019 |
| 2025.06.23 | 00.08 | Adopted changes on NIST-0018 and NIST-0019 due to the ongoing review |
| 2025.07.09 | 01.00 | Published as Final version 01.00 |
| 2025.09.29 | 01.01 | Merged NEC.AO-0063, NEC-0064, NOK.AO-0308, NOK-0326 |
| 2025.10.13 | 01.02 | Fixed typo introduced through NEC.AO-0063, merged NOK.AO-0336 |
| 2025.10.14 | 01.03 | notes in table 5-1: space -> tab; R004 -> R005; merge some of the cells in table 5-1 |
| 2025.11.19 | 01.04 | set copyright year to 2026, aligned references to OFH-related documents in table 5-1 |
| 2025.11.25 | 02.00 | Published as Final version 02.00 |