O-RAN.WG11.TR.SCM.0-R005-v02.00

Technical Report

O-RAN Work Group 11 (Security work group) Study on Security Configuration Management

Copyright © 2026 by the O-RAN ALLIANCE e.V. The copying or incorporation into any other work of part or all of the material available in this document in any form without the prior written permission of O-RAN ALLIANCE e.V. is prohibited, save that you may print or download extracts of the material of this document for your personal use, or copy the material of this document for the purpose of sending to individual third parties for their information provided that you acknowledge O-RAN ALLIANCE as the source of the material and that you inform the third party that these conditions apply to them and that they must comply with them. O-RAN ALLIANCE e.V., Buschkauler Weg 27, 53347 Alfter, Germany Register of Associations, Bonn VR 11238, VAT ID DE321720189

- Contents

List of figures	3

List of tables	3

Foreword	4

Modal verbs terminology	4

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definition of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	SCM Background	6

4.1	Introduction	6

4.2	Considerations	7

5	Study and identify the security gaps related to SCM	7

5.1	Assets	7

5.1.1   	 O-RAN Architecture Elements	7

5.1.2   	 O-RAN interfaces	7

5.2	Security Controls	8

5.3	Input recommendations	8

5.4	Threat template	8

5.5	SCM Threats analysis	8

5.5.1   	  Certificate related threats	9

5.5.2   	  NACM related threats	10

5.5.3   	  LDAP related threats	11

6	Study and recommendation related to SCM	12

7	Conclusion	12

Annex: Change history/Change request (history)	13

# List of figures

No table of figures entries found.

# List of tables

No table of figures entries found.

# Foreword

This Technical Report (TR) has been produced by WG11 of the O-RAN ALLIANCE.

The content of the present document is subject to continuing work within O-RAN and may change following formal O-RAN approval. Should the O-RAN Alliance modify the contents of the present document, it will be re-released by O-RAN with an identifying change of version date and an increase in version number as follows:

version xx.yy.zz

where:

xx:	the first digit-group is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc. (the initial approved document will have xx=01).  Always 2 digits with leading zero if needed.

yy:	the second digit-group is incremented when editorial only changes have been incorporated in the document. Always 2 digits with leading zero if needed.

zz:	the third digit-group included only in working versions of the document indicating incremental changes during the editing process. External versions never include the third digit-group.  Always 2 digits with leading zero if needed.

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the O-RAN Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in O-RAN deliverables except when used in direct citation.

# Scope

The main objective for this work item is to identify and develop the security management YANG models for O-RAN framework fulfilling the security requirements specified in [i.2]. The specification will be based on the YANG data modeling language (see RFC 7950). Security YANG models in O-RAN will apply to relevant O-RAN interfaces (O1, Open Fronthaul M-plane). This work item will perform the following steps,

- Identify if there is any existing security management YANG model available in O-RAN specifications

- Study and recommend the security requirement for YANG model aligning with [i.2]

- Study and identify the gaps related to security configuration

- Study the YANG models for certificate configuration

- Study the YANG models for NETCONF access control such as NACM configuration

- Study the YANG models for LDAP configuration

# References

## Normative references

Not applicable.

## Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies. In the case of a reference to a 3GPP document, a non-specific reference implicitly refers to the latest version of that document in Release 18, or the latest 3GPP release prior to Release 18 that includes that document.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, O-RAN cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document, but they assist the user with regard to a particular subject area.

- O-RAN.WG11.TS.SecProtSpec: "Security Protocols Specification".

- O-RAN.WG11.TS.SRCS: "Security Requirements and Controls Specification".

- O-RAN.WG11.TR.Threat-Modeling: "O-RAN Security Threat Modelling and Risk Assessment".

- O-RAN.WG11.TS.STS: "O-RAN Security Test Specifications".

- O-RAN.WG4.TS.MP: "Management Plane Specification".

- IETF RFC 7950: "The YANG 1.1 Data Modeling Language".

- O-RAN.WG10.TS.Information Model and Data Models: "O-RAN Information Model and Data Models Specification".

- O-RAN.WG10.TS.O1NRM: "O1 Network Resource Model".

- O-RAN.WG4.TS.CTI-TMP: " Cooperative Transport Interface Transport Management Procedures Specification".

- O-RAN.WG5.O-DU-O1: "O1 Interface specification for O-DU".

- O-RAN-WG5.O-CU-O1: "O1 Interface specification for O-CU-UP and O-CU-CP".

- An extensible YANG validator and converter in python, https://github.com/mbj4668/pyang.

- O-RAN.WG1.TS.OAD: "O-RAN Architecture Description".

- O-RAN.WG10.O1-Interface: "O-RAN Operations and Maintenance Interface Specification".

# Definition of terms, symbols and abbreviations

## Terms

For the purposes of the present document, the terms [given in [i.1] and the following] apply:

Open Fronthaul M-Plane: Management interface controlling the O-RU, generally driven from the O-DU but in the case of the hybrid topology also driven from the SMO as specified in [i.13].

NOTE 1: Please refer to [i.5] for more details.

O1: Interface used by the SMO framework, as specified in Clause 5.3.1, for operation and management of O-RAN NFs as specified in [i.13] [i.14].

O-Cloud: Cloud platform that provides O-RAN standardized interfaces, hosting O-RAN defined software components.

SMO: A Service Management and Orchestration framework.

NOTE 2: Please refer to Clause 5.3.1 for more details as specified in [i.13].

## Symbols

Void

## Abbreviations

For the purposes of the present document, the abbreviations [given in [i.1]and the following] apply:

LDAP 	Lightweight directory access protocol

NACM 	NETCONF Access Control Model

NETCONF	Network Configuration Protocol

NF		Network Function

O-RAN	Open Radio Access Network

O-Cloud	O-RAN Cloud

PNF		Physical Network Function

SMO		Service Management and Orchestration

VNF		Virtualised Network Function

YANG	Yet Another Next Generation

# SCM Background

## Introduction

Management of security configuration is currently not handled in a consistent way in O-RAN. Different work groups see the topic in different ways. In some cases, this is expected to be a topic only for the security work group while in other cases it is defined within a work group.

For example, there are YANG models related to NACM included in WG4 definitions for Open Fronthaul M-plane. For O1, there are security requirements on NACM stated in [i.2]. There may be other areas where security configuration such as LDAP configuration, NACM configuration, Certificate Management configuration is required. Some of the security requirements related to NACM, LDAP are defined in [i.2].

There are several discussions on O1 security configuration aspects such as NACM, etc happening across O-RAN WGs (WG11, W10, etc) showing a need to have Security Configuration Management specifications in O-RAN.

## Considerations

This document makes the following considerations:

- This TR performs a detailed study of the Security Configuration Management for O1 interface and Open Fronthaul M-plane.

- The existing O-RAN specifications such as  O-RAN.WG10.Information Model and Data Models [i.7], O-RAN.WG10.O1NRM.1 [i.8], O-RAN.WG4.MP-YANGs [i5], O-RAN.WG4.CTI-TMP-YANG [i.9], O-RAN.WG5.O-DU-O1.1 [i.10], O-RAN.WG5.O-CU-O1.0 [i.11] discuss the YANG models which are considered as the baseline specifications for the Security Configuration Management TR to identify and develop the security YANG models for O-RAN framework fulfilling the security requirements specified in [i.2].

- The Security YANG model considers the rules and guideline as specified in O-RAN specifications [i.5] [i.7] [i.8] [i.9] [i.10] [i.11].

- The pyang YANG validator tool is considered for evaluating the security YANG model as recommended in [i.12].

- The recommended security YANG models for O1 interface provided in this report to be shared with the impacted O-RAN Alliance working group, such as WG4 and WG10.

# Study and identify the security gaps related to SCM

## Assets

This document makes the following considerations:

## 5.1.1   	 O-RAN Architecture Elements

- ASSET-C-01: Logical module: Service Management and Orchestration (SMO)

- ASSET-C-02: Near-RT RIC software

- ASSET-C-03: O-CU-CP software

- ASSET-C-04: O-CU-UP software

- ASSET-C-05: O-DU software

- ASSET-C-06: O-RU software

- ASSET-C-07: O-eNB

- ASSET-C-31: Shared O-RU

## 5.1.2   	 O-RAN interfaces

- ASSET-C-22: O1 interface, including protocol stack

- ASSET-C-24: Open FH M-plane, including protocol stack

## Security Controls

The following security controls should be considered for the security gap analysis:

- Control-1: Certificate Management

- Control-2: TLS

- Control-3: mTLS

- Control-4: SSH

- Control-5: NACM with NETCONF

- Control-6: MACsec

- Control-7: IPsec

- Control-8: 802.1X

## Input recommendations

The following are considered for security configuration management gap analysis:

- Certificate configuration

- NETCONF access control, NACM configuration

- LDAP configuration

## Threat template

| Threat ID | O-RAN threat ID |
| --- | --- |
| Threat title | [Threat Title will include suitable reference of 3GPP/IETF/ETSI] |
| Threat description | Introduction about the threat |
| Threat type | Spoofing Tampering Repudiation Information disclosure Denial of Service Elevation of Privilege |
| Vulnerability | TBD |
| Impact type | Authenticity Integrity Non-repudiation Confidentiality Availability Authorization |
| Affected Assets | Asset identification |

## SCM Threats analysis

| Threat ID | T-SCM-GEN-01 |
| --- | --- |
| Threat title | Exploitation due to misconfigured security related configuration parameters |
| Threat description | A malicious actor may be able to change the security related configuration parameters of an O-RAN architectural element if administrator has not configured the parameters appropriately. |
| Threat type | Elevation of privilege |
| Vulnerability | Misconfiguration of security related configuration parameters. |
| Impact type | Authorization |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

| Threat ID | T-SCM-GEN-02 |
| --- | --- |
| Threat title | Excessive NETCONF operation requests |
| Threat description | A malicious actor may be able to attempt excessive NETCONF operation requests to an O-RAN architectural element which may lead to service disruption. |
| Threat type | DoS |
| Vulnerability | Insufficient support to detect and handle excessive NETCONF operation requests |
| Impact type | Availability |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

## 5.5.1   	  Certificate related threats

| Threat ID | T-SCM-Cert-01 |
| --- | --- |
| Threat title | Configuration of incorrect Certificate parameter |
| Threat description | Certificate parameters may be misconfigured intentionally or unintentionally which may lead to the CA issuing a rogue certificate or the wrong certificate |
| Threat type | Denial of Service, Tampering |
| Vulnerability | It may lead to incorrect certificate being issued by the CA issuer |
| Impact type | Availability |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

| Threat ID | T-SCM-Cert-02 |
| --- | --- |
| Threat title | Tampering of Certificate parameter |
| Threat description | If the certificate parameters are tampered or comprised by the malicious actor, which may lead to initiation of valid certificate request to CA by the attacker |
| Threat type | Denial of Service, Tampering |
| Vulnerability | Attackers may claim as trusted entity |
| Impact type | Confidentiality Authenticity Availability |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

## 5.5.2   	  NACM related threats

| Threat ID | T-SCM-NACM-01 |
| --- | --- |
| Threat title | Exploitation of incorrectly implemented default access parameters for NACM |
| Threat description | A malicious actor may be able to access and perform NACM related configuration changes to an O-RAN architecture element if the vendor has not correctly implemented the default access parameters. |
| Threat type | Elevation of privilege |
| Vulnerability | Incorrect implementation of default access parameters |
| Impact type | Authorization |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

| Threat ID | T-SCM-NACM-02 |
| --- | --- |
| Threat title | Unauthorized access to NACM configuration |
| Threat description | A malicious actor may be able to access NACM configuration which may allow the malicious actor to perform unauthorized operation in an O-RAN architecture element. |
| Threat type | Elevation of privilege |
| Vulnerability | Insufficient access protection for NACM configuration |
| Impact type | Authorization |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

| Threat ID | T-SCM-NACM-03 |
| --- | --- |
| Threat title | Unauthorized NETCONF operation requests |
| Threat description | A malicious actor may be able to perform the NETCONF operation requests or data store write requests without legitimate access rights to the O-RAN architecture elements. |
| Threat type | Elevation of privilege |
| Vulnerability | Improper authorization process and lack of re-validating the authorization requests |
| Impact type | Authorization |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

## 5.5.3   	  LDAP related threats

| Threat ID | T-SCM-LDAP-01 |
| --- | --- |
| Threat title | Abuse of LDAP configuration information |
| Threat description | A malicious actor may attempt to access the LDAP configuration information in an O-RAN architectural element and perform unauthorized modification of configuration information (i.e. delete data). |
| Threat type | Elevation of privilege |
| Vulnerability | Insufficient protection of LDAP configuration information |
| Impact type | Authorization |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

| Threat ID | T-SCM-LDAP-02 |
| --- | --- |
| Threat title | Compromise of LDAP configuration by illegitimate actor |
| Threat description | An illegitimate actor may attempt to compromise an O-RAN architectural element by modifying the LDAP service account used for LDAP binding. |
| Threat type | Elevation of privilege |
| Vulnerability | Insufficient validation for the legitimate actor |
| Impact type | Authorization |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

| Threat ID | T-SCM-LDAP-03 |
| --- | --- |
| Threat title | Compromise of LDAP session credentials |
| Threat description | A malicious actor may steal LDAP session credentials if an O-RAN architectural element is not configured correctly for securing the LDAP communication. |
| Threat type | Information Disclosure |
| Vulnerability | Misconfiguration related to secure LDAP communication |
| Impact type | Confidentiality |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

| Threat ID | T-SCM-LDAP-04 |
| --- | --- |
| Threat title | Repeated failed operation while using valid service account |
| Threat description | A malicious actor may use reusable LDAP service account of an O-RAN architectural element and attempt to perform unauthorized operations repeatedly. This may lead to service disruption and necessary configuration changes in the O-RAN architecture element. |
| Threat type | Denial of service |
| Vulnerability | Insufficient protection for credentials |
| Impact type | Availability |
| Affected Asset | Logical module: Service Management and Orchestration (SMO) [ASSET-C-01], Near-RT RIC [ASSET-C-02], O-CU-CP [ASSET-C-03], O-CU-UP [ASSET-C-04], O-DU [ASSET-C-05], O-RU [ASSET-C-06], O-eNB [ASSET-C-07], Shared O-RU [ASSET-C-31] |

# Study and recommendation related to SCM

Editor’s note: This clause will study and recommend the security requirements and security usecases related to security configuration management like NACM, Certificate and LDAP aligning with SRCS and SPS.

Void

# Conclusion

Editor’s note : This clause will provide a summary for the SCM TR.

Void

# Annex:
Change history/Change request (history)

| Date | Revision | Description |
| --- | --- | --- |
| 2025.01.28 | 00.00.01 | Skeleton proposal |
| 2025.03.11 | 00.02 | Approved CR in the month of feb and March 2025- Internal O-RAN release |
| 2025.03.12 | 00.03 | Feedback received during the SCM WI TR meeting [4] on 11/03/2025 |
| 2025.04.16 | 00.04 | Approved CR in the month of April 2025 during the SCM WI TR meeting |
| 2025.06.03 | 00.05 | Approved CR in the month of June 2025 during the SCM WI TR meeting |
| 2025.06.18 | 00.06 | Updated the TR based on comments received and CR approved till June 2025 ERI-2025.02.11-WG11-CR0207-SCM-Clause1_Scope-v02.docx ERI-2025.02.11-WG11-CR0208-SCM-Clause2_Reference-v02.docx ERI-2025.02.24-WG11-CR0213-SCM-Clause3.1_Terms-v03.docx ERI-2025.02.24-WG11-CR0214-SCM-Clause3.3_Abbreviations-v01.docx ERI-2025.02.25-WG11-CR0211-SCM-Clause4_Background-v03.docx ERI-2025.02.25-WG11-CR0212-SCM-Clause4.1_Consideration-v03.docx ERI-2025.04.09-WG11-CR0226-SCM-Clause5.1_assets-v03.docx ERI-2025.04.09-WG11-CR0227-SCM-Clause5.2_SecurityControls-v04.docx ERI-2025.04.09-WG11-CR0228-SCM-Clause5.3_ Recommendation for Security Configuration Management-v03.docx ERI-2025.05.07-WG11-CR0238-SCM-Clause5.4_ Threats Template-v01.docx ERI-2025.05.07-WG11-CR0239-SCM-Clause5.5_ SCMThreatAnalysis-v01.docx ERI-2025.05.21-WG11-CR0244-SCM-Clause5.5.1-CertificateRelatedThreats-v02.docx |
| 2025.10.30 | 01.01 | Updated the TR based on comments received and CR approved till October 2025 ERI.AO-2025.08.12-WG11-CR0250-SCM-Clause5.5.2-NACMrelatedthreats-v06.docx ERI.AO-2025.08.12-WG11-CR0251-SCM-Clause5.5.3-LDAPrelatedthreats-v06.docx ERI.AO-2025.08.27-WG11-CR0257-SCM-Clause5.5.3-LDAPrelatedthreats_2-v06.docx ERI.AO-2025.09.09-WG11-CR0261-SCM-Clause5.5-Configurationthreat-v05.docx NEC-2025.10.22-WG11-CR-0072-SCM-ReferenceUpdate-v01.docx |