O-RAN.WG5.TS.D2AP-0.R005-v01.00

Technical Specification

O-RAN Work Group 5 ( Open F1/W1/E1/X2/Xn/D2 interfaces) D2 Interface: D2 Application Protocol (D2AP)

Copyright © 2025 by the O-RAN ALLIANCE e.V. The copying or incorporation into any other work of part or all of the material available in this specification in any form without the prior written permission of O-RAN ALLIANCE e.V. is prohibited, save that you may print or download extracts of the material of this specification for your personal use, or copy the material of this specification for the purpose of sending to individual third parties for their information provided that you acknowledge O-RAN ALLIANCE as the source of the material and that you inform the third party that these conditions apply to them and that they must comply with them. O-RAN ALLIANCE e.V., Buschkauler Weg 27, 53347 Alfter, Germany Register of Associations, Bonn VR 11238, VAT ID DE321720189

- Contents

Contents	2

List of figures	4

List of tables	4

Foreword	5

Modal verbs terminology	5

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	7

3	Definition of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Interface principles and characteristics	9

4.1	Procedure specification principles	9

4.2	Forwards and backwards compatibility	9

4.3	Specification notations	9

5	D2 Control Plane protocol	10

5.1	D2CP services	10

5.2	D2CP Functions	10

5.3	D2CP procedures	10

5.3.1	Interface Management procedures	10

5.3.2	UE Management procedures	14

5.4	Elements of D2CP Communication	18

5.4.1	General	18

5.4.2	Message Functional Definition and Content	19

5.4.3	Information Element Definitions	33

5.4.4	Message and Information Element Abstract Syntax (with ASN.1)	44

5.4.5	Message Transfer Syntax	66

5.4.6	Timers	66

6	D2 User Plane protocol	67

6.1	D2UP services	67

6.2	D2UP Functions	67

6.3	D2UP Procedures	67

6.3.1	Downlink data transfer	67

6.3.2	UCI Transfer	71

6.4	Elements of D2UP Communication	72

6.4.1	General	72

6.4.2	Message Format for the D2UP	72

6.4.3	Coding of information elements in frames	77

Annex A (informative): Examples for DL SCHEDULING REQUEST	85

A.1: 	New Data transmission:	85

A.2:	Retransmission	87

A.3:	New Data transmission and retransmission (Both)	88

Annex (informative):  Change history/Change request (history)	90

- List of figures

Figure 5.3.1.1-1: D2 Setup, successful operation	11

Figure 5.3.1.1-2: D2 Setup, unsuccessful operation	11

Figure 5.3.1.2-1: D2 Configuration Update, successful operation	12

Figure 5.3.1.2-2: D2 Configuration Update, unsuccessful operation	12

Figure 5.3.1.3-1: D2 Release	13

Figure 5.3.1.4-1: D2 Error Indication initiated by O-DU1	13

Figure 5.3.1.4-2: D2 Error Indication initiated by O-DU2	14

Figure 5.3.2.1-1: UE Addition Procedure	14

Figure 5.3.2.2-1: UE Modification Update procedure	16

Figure 5.3.2.3-1: UE Modification Indication procedure	17

Figure 5.3.2.4-1: UE Reconfiguration Complete procedure	18

Figure 5.3.2.5-1: UE Deletion procedure	18

Figure 6.3.1.1-1: DL scheduling request	68

Figure 6.3.1.2-1: DL data required	69

Figure 6.3.1.3-1: DL data transfer	70

Figure 6.3.1.4-1: PUCCH resource transfer	70

Figure 6.3.2-1: UCI transfer	71

Figure 6.4.1-1: Example frame format for D2UP	72

Figure 6.4.2-1: D2-u header	73

Figure 6.4.2.1-1: DL SCHEDULING REQUEST message format	74

Figure 6.4.2.2-1: DL DATA REQUIRED message format	75

Figure 6.4.2.3-1: DL DATA TRANSFER message format	75

Figure 6.4.2.4-1: PUCCH RESOURCE TRANSFER message format	76

Figure 6.4.2.5-1: UCI TRANSFER message format	76

Figure A.1-1: DL SCHEDULING REQUEST with newTx with SlotType as SlotRange	86

Figure A.1-2: DL SCHEDULING REQUEST with newTx with SlotType as SlotMask	87

Figure A.2-1: DL SCHEDULING REQUEST for retransmission	88

Figure A.3-1: DL SCHEDULING REQUEST for transmission and retransmission (Both)	90

- List of tables

Table 6.4.3.3-1: Message ID	76

- Foreword

This Technical Specification (TS) has been produced by WG5 of the O-RAN Alliance.

The content of the present document is subject to continuing work within O-RAN and may change following formal O-RAN approval. Should the O-RAN Alliance modify the contents of the present document, it will be re-released by O-RAN with an identifying change of version date and an increase in version number as follows:

version xx.yy.zz

where:

xx:	the first digit-group is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc. (the initial approved document will have xx=01).  Always 2 digits with leading zero if needed.

yy:	the second digit-group is incremented when editorial only changes have been incorporated in the document. Always 2 digits with leading zero if needed.

zz:	the third digit-group included only in working versions of the document indicating incremental changes during the editing process. External versions never include the third digit-group.  Always 2 digits with leading zero if needed.

- Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the O-RAN Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in O-RAN deliverables except when used in direct citation.

Scope

The focus of the present document is to specify the stage 3 details of the D2 interface for supporting inter O-DU NR carrier aggregation.  The present specification covers the procedures, messages and IEs for establishment and functioning of D2 interface that includes D2 interface bring up procedures, UE addition and configuration, control and user plane information exchange across O-DUs.

References

Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, O-RAN cannot guarantee their long-term validity.

The following referenced documents are necessary for the application of the present document.

[1]	O-RAN.WG1.TS.OAD: "O-RAN Architecture Description"

[2]		O-RAN.WG5.TS.D2GAP: "D2 Interface: General Aspects and Principles".

[3]		3GPP TS 38.413 V18.6.0: "NG-RAN; NG Application Protocol (NGAP)".

[4] 		3GPP TS 38.401 V18.6.0: "5G; NG-RAN; Architecture description ".

[5]		3GPP TS 38.331 V18.6.0: "5G; NG-RAN; Radio Resource Control (RRC)".

[6]		3GPP TS 38.473 V18.6.0: "5G; NG-RAN; F1 Application Protocol (F1AP)".

[7]		3GPP TS 38.133 V18.10.0: "5G; NG-RAN; Requirements for support of radio resource management".

[8]		3GPP TS 25.921:	 "Guidelines and principles for protocol description and error handling".

[9]		IETF RFC 4960:	"Stream Control Transmission Protocol".

[10]		IETF RFC 8200 (2017-07): 	"Internet Protocol, Version 6 (IPv6) Specification".

[11]	IETF RFC 791 (1981-09): "Internet Protocol".

[12] 		ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules -

Specification of Packed Encoding Rules (PER) ".

[13] 		ITU-T Recommendation X.680 (2002-07): "Information technology – Abstract Syntax Notation

One (ASN.1): Specification of basic notation".

[14] 		ITU-T Recommendation X.681 (2002-07): "Information technology – Abstract Syntax Notation

One (ASN.1): Information object specification".

[15] 	3GPP TS 23.003 V18.6.0: "Numbering, addressing and identification".

[16]	3GPP TS 23.501 V18.11.0: "System Architecture for the 5G System".

[17]	3GPP TS 38.213 V18.7.0: “Physical layer procedures for control”.

[18]	3GPP TS 38.212 V18.7.0: “Physical layer procedures for control”.

[19]	3GPP TS 29.281 V18.4.0:	"General Packet Radio System (GPRS) Tunnelling Protocol User Plane

(GTPv1-U)".

[20]	3GPP TS 38.321 V18.7.0:	“Medium Access Control (MAC) protocol specification”.

Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, O-RAN cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document, but they assist the user with regard to a particular subject area.

[i.1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

Definition of terms, symbols and abbreviations

Terms

For the purposes of the present document, the terms given in [i.1] and the following apply:

carrier aggregation: aggregation of two or more NR or E-UTRA component carriers in order to support wider transmission bandwidths

PCell O-DU:  O-DU, where the PCell is configured for a UE to support inter O-DU carrier aggregation.

pO-DU:  O-DU, where the PCell is configured for a UE to support inter O-DU carrier aggregation. This is same as PCell O-DU used in the D2 GAP specifications.

SCell O-DU:  O-DU, where only SCell(s) are configured for a UE to support inter O-DU carrier aggregation.

sO-DU:  O-DU, where only SCell(s) are configured for a UE to support inter O-DU carrier aggregation. This is same as SCell O-DU used in the D2 GAP specifications.

Elementary Procedure: D2CP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between the O-DUs. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. An EP consists of an initiating message and  possibly a response message.

Two kinds of EPs are used:

- Class 1: Elementary Procedures with response (success and/or failure).

- Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

- A signaling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

- A signaling message explicitly indicates that the EP failed.

- On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

- One signaling message reports both a successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

Symbols

For the purposes of the present document, the symbols [given in [i.1] and the following] apply:

NRB	Transmission bandwidth configuration, expressed in units of resource blocks (for E-UTRA) <example symbol definition>

Abbreviations

For the purposes of the present document, the abbreviations given in [i.1] and the following] apply:

3GPP		3rd Generation Partnership Project

ASN.1		Abstract Syntax Notation One

CM		Configuration Management

D2AP		D2 Application Protocol

D2-c		D2 Control plane interface

D2CP	D2 Control Protocol

D2-u		D2 User Plane interface

D2UP	D2 User Plane Protocol

gNB		gNodeB (applies to NR)

IP		Internet Protocol

MAC		Medium Access Control

NF		Network Function

NGRAN		Next Generation Radio Access Network

NR		New Radio

O-CU-CP		O-RAN Central Unit – Control Plane.

O-CU-UP		O-RAN Central Unit – User Plane

O-DU		O-RAN Distributed Unit

O-RAN		Open Radio Access Network

O-RU		O-RAN Radio Unit

PCell	Primary Cell

RAN		Radio Access Network

RLC 		Radio Link Control

SCell	Secondary Cell

SCTP 		Stream Control Transmission Protocol

SMO		Service Management and Orchestration

TR		Technical Report

TRS	Trace Recording Session

TS		Technical Specification

UE		User Equipment

Interface principles and characteristics

Procedure specification principles

The principle for specifying the procedure logic is to specify the functional behaviour of the terminating node exactly and completely. Any rule that specifies the behaviour of the originating node shall be possible to be verified with information that is visible within the system.

The following specification principles have been applied for the procedure text in clause 5.3

-	The procedure text discriminates between:

- Functionality which "shall" be executed

The procedure text indicates that the receiving node "shall" perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the initiating message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

- Functionality which "shall, if supported" be executed

The procedure text indicates that the receiving node "shall, if supported," perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included.

Forwards and backwards compatibility

The forward and backward compatibility of the D2CP protocol is assured by a mechanism where all current and future messages, and IEs or groups of related IEs, include ID and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

For D2UP protocol extension fields are added in the messages to support forward and backward compatibility.

Specification notations

For the purposes of the present document, the following notations apply:

Procedure 		When referring to an elementary procedure in the specification the Procedure Name is written with

the first letters in each word in upper case characters followed by the word "procedure", e.g.

D2 Setup procedure.

Message 			When referring to a message in the specification the MESSAGE NAME is written with all letters

in upper case characters followed by the word "message", e.g. D2 SETUP REQUEST message.

IE 					When referring to an information element (IE) in the specification the Information Element Name

is written with the first letters in each word in upper case characters and all letters in Italic font

followed by the abbreviation "IE", e.g. D2 UE ID IE.

Value of an IE 	When referring to the value of an information element (IE) in the specification the "Value" is

written as it is specified in the specification enclosed by quotation marks, e.g. "Value".

D2 Control Plane protocol

D2CP services

D2CP provides the signalling services between the peer O-DUs which are required to fulfil the D2CP functions described in clause 5.2. D2CP services are divided into two groups:

Non UE-associated services:	They are related to the whole D2 interface instance between the O-DUs utilising a non UE-associated signalling connection.

UE-associated services:	They are related to a UE. D2CP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UEs in question.

Unless explicitly indicated in the procedure specification, at any instance in time one protocol endpoint shall have a

maximum of one ongoing D2CP procedure related to a certain UE.

D2CP Functions

D2CP functions are defined in O-RAN.WG5.TS.D2GAP[2].

D2CP procedures

Interface Management procedures

D2 Setup

The purpose of the D2 Setup procedure is to establish connection with peer O-DUs and exchange the cell level information to interoperate on the D2 interface. This procedure is the first D2 procedure triggered for the D2-C interface after a TNL association has become operational.

The procedure uses non-UE associated signalling.

O-DU2

O-DU1

D2 SETUP REQUEST

D2 SETUP RESPONSE

This procedure erases any existing D2 related application data associated with the pair of O-DUs in both the nodes and replaces it by the one received. This procedure also re-initialises the D2 UE-related contexts (if any) and erases all related signalling connections in the two nodes.

Figure 5.3.1.1-1: D2 Setup, successful operation

As shown in Figure 5.3.1.1-1, the O-DU1 initiates the procedure by sending a D2 SETUP REQUEST message including the cell information to the O-DU2. The O-DU2 responds with a D2 SETUP RESPONSE message including the cell information.

The received Served Cell List IE in D2 SETUP REQUEST and D2 SETUP RESPONSE messages contains the list of respective SCells for inter O-DU CA. It is stored in receiving node and used for the duration of the operational TNL association. After successful completion of this procedure, the D2 interface is operational and other D2 messages may be exchanged.

The O-DU1 and O-DU2 may share Start CRNTI and End CRNTI for the UEs in D2 SETUP REQUEST and D2 SETUP RESPONSE messages.

The O-DUs exchange the D2U Info List IE containing the information for D2-U communication. IP Address and UDP port number to be used for D2-U message exchange in respective O-DUs is specified in D2 SETUP REQUEST and D2 SETUP RESPONSE messages. Receiver O-DU shall use the received IP address and UDP port number while sending D2-U messages to other O-DU.

D2U PV List IE, received in D2 SETUP REQUEST message indicates the D2 protocol version supported by O-DU1. If O-DU2 supports any protocol version from the list in descending order, it shall include the selected protocol version in D2U PV List IE while sending D2 SETUP RESPONSE message.

O-DU2

O-DU1

D2 SETUP REQUEST

D2 SETUP FAILURE

The O-DUs stores the T_clearTB timer value exchanged in the D2 SETUP REQUET and D2 SETUP RESPONSE messages. This timer is used during D2-U procedures. Receiver node shall use T_ClearTB IE when it is operating as sO-DU for the UEs.

Figure 5.3.1.1-2: D2 Setup, unsuccessful operation

If the O-DU2 cannot accept the setup, it shall respond with a D2 SETUP FAILURE message with Cause IE filled with appropriate cause as shown in Figure 5.3.1.1-2.

If the O-DU2 does not support any of the protocol version indicated in the D2U PV List IE, procedure will be considered as failure, and D2 SETUP FAILURE message shall be sent to O-DU1 with cause set as “Protocol Version Not supported”.

If the D2 SETUP FAILURE message includes the Time to wait IE, the O-DU1 shall wait at least for the indicated time before reinitiating the D2 setup towards the O-DU2.

If the O-DU1 does not receive either D2 SETUP RESPONSE message or D2 SETUP FAILURE message, the O-DU1 may reinitiate the D2 Setup procedure towards the same O-DU node, provided that the content of the new D2 SETUP REQUEST message is identical to the content of the previously unacknowledged D2 SETUP REQUEST message.

If the O-DU1 receives a D2 SETUP REQUEST message from the peer entity on the same D2 interface:

- In case the O-DU1 answers with a D2 SETUP RESPONSE message and receives a subsequent D2 SETUP FAILURE, the O-DU1 node shall consider the D2 interface as non operational and the procedure as unsuccessfully terminated.

- In case the O-DU1 answers with a D2 SETUP FAILURE message and receives a subsequent D2 SETUP RESPONSE message, the O-DU1 shall ignore the D2 SETUP RESPONSE message and consider the D2 interface as non operational.

D2 Configuration Update

The purpose of the D2 Configuration update procedure is to update the cell configuration in peer O-DU. This procedure may be triggered by any O-DU in case of modification in configuration of existing cells, release of any existing cell or addition of new cell. The procedure uses non-UE associated signalling.

In Figure 5.3.1.2-1, O-DU1 is shown as the triggering node for successful completion of configuration update procedure.

O-DU2

O-DU1

D2 CONFIGURATION UPDATE

D2 CONFIGURATION UPDATE ACKNOWLEDGE

Figure 5.3.1.2-1: D2 Configuration Update, successful operation

The O-DU1 initiates the procedure by sending a D2 CONFIGURATION UPDATE message to the O-DU2 with the configuration that needs to be updated. The O-DU2 responds with a D2 CONFIGURATION UPDATE ACKNOWLEDGE message to confirm that it has successfully updated the configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the Served Cell To Add Item IE is contained in the D2 CONFIGURATION UPDATE message, the O-DU2 shall store the information for newly added cell.

If the Served Cells To Modify Item IE is contained in the D2 CONFIGURATION UPDATE message, the O-DU2 shall modify information of existing cell indicated by Old NR CGI IE according to the updated information received in the message.

If the Served Cells To Delete Item IE is contained in the D2 CONFIGURATION UPDATE message, the O-DU2 shall delete the cell information indicated by Old NR CGI IE.

The responding node, O-DU2 shall include, within the Updated Cells Item IE of the D2 CONFIGURATION UPDATE ACKNOWLEDGE message, all cells that were listed in the Served Cell To Add Item and Served Cells To Modify Item IEs of the D2 CONFIGURATION UPDATE message.  For each cell, the Result IE within the Updated Cells Item IE shall indicate the outcome of the configuration update procedure, explicitly denoting success or failure.

Cells specified in the Served Cells To Delete Item IE of the D2 CONFIGURATION UPDATE message shall not be included in the D2 CONFIGURATION UPDATE ACKNOWLEDGE message.

O-DU2

O-DU1

D2 CONFIGURATION UPDATE

D2 CONFIGURATION UPDATE FAILURE

Figure 5.3.1.2-2: D2 Configuration Update, unsuccessful operation

If the O-DU2 cannot accept the update, or if reconfiguration fails for all the cells, it shall respond with a D2 CONIFGURATION UPDATE FAILURE message with Cause IE filled with appropriate cause as shown in Figure 5.3.1.2-2.

If the D2 CONIFGURATION UPDATE FAILURE message includes the Time to wait IE, the O-DU1 shall wait at least for the indicated time before reinitiating the D2 CONIFGURAITON UPDATE message towards the same O-DU2.

D2 Release

The purpose of the D2 Release procedure is to trigger the release of the D2 connection between a pair of O-DUs and erase all D2 application-level data.

The procedure uses non-UE-associated signaling.

In Figure 5.3.1.3-1, O-DU1 is shown as the triggering node.

O-DU2

O-DU1

D2 RELEASE REQUEST

D2 RELEASE RESPONSE

Figure 5.3.1.3-1: D2 Release

O-DU1 initiates the procedure by sending the D2 RELEASE REQUEST message to O-DU2. O-DU1 shall include the cause IE that triggered the release of D2 connection. Upon reception of the D2 RELEASE REQUEST message the O-DU2 shall remove all resources associated with that D2 connection and all D2 application-level data. The O-DU2 shall reply with the D2 RELEASE RESPONSE message. After receiving the D2 RELEASE RESPONSE message, the O-DU1 shall initiate removal of the TNL association towards O-DU2 and shall remove all resources associated with that D2 connection and all D2 application-level data.

Error Indication

The Error Indication procedure is initiated by an D2 ERROR INDICATION message sent from the receiving node,

in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

Figure 5.3.1.4-1 indicates the Error Indication procedure initiated by O-DU1. It can be initiated by O-DU2 as shown in Figure 5.3.1.4-2 in case error is detected on the reception of message from O-DU2.

O-DU1

O-DU2

D2 ERROR INDICATION

Figure 5.3.1.4-1: D2 Error Indication initiated by O-DU1

O-DU1

O-DU2

D2 ERROR INDICATION

Figure 5.3.1.4-2: D2 Error Indication initiated by O-DU2

When the conditions as mentioned below are fulfilled, the Error Indication procedure is initiated by the D2 ERROR

INDICATION message sent from the receiving node. The D2 ERROR INDICATION message shall contain the Cause IE

- In case of Abstract Syntax Error, the cause shall be set to “Abstract Syntax Error”.  The Transaction ID IE shall indicate the message that triggered the Error scenario.

- In case of Logical Error in a response message of a Class 1 procedure, or failure to comprehend Transaction ID IE from a received message, the procedure shall be considered as unsuccessfully terminated or not terminated (e.g., transaction ID unknown in response message), and local error handling shall be initiated

UE Management procedures

UE Addition

The purpose of the UE Addition procedure, as shown in Figure 5.3.2.1-1, is to establish the D2 UE context in the sO-DU by adding one or more of its cells as SCells for the UEs mentioned in the UE ADDITION REQUEST message. The procedure uses UE-associated signalling. The pO-DU initiates the procedure by sending UE ADDITION REQUEST message to sO-DU if it is able to allocate PUCCH resources to receive the CSI information for new SCell. sO-DU responds back with UE ADDTION RESPONSE message with the information on success or failure of SCell(s) addition for the UE(s) mentioned in the message.

sO-DU

pO-DU

UE ADDITION REQUEST

UE ADDITION RESPONSE

Figure 5.3.2.1-1: UE Addition Procedure

If sO-DU accepts the request for at least one of candidate SCell(s) listed in SCell To Be Added List IE for the UE(s) present in UE To Be Added Item IE, it establishes the D2 UE context for the UE(s) and stores the D2 UE ID and C-RNTI. Association between a UE and D2 UE ID does not change unless D2 UE context is deleted.

If the DRX-Config IE is contained in the UE ADDITION REQUEST message, the sO-DU shall use the provided value from the pO-DU.

If the MeasGapConfig IE is contained in the UE ADDITION REQUEST message, the sO-DU shall use the provided value from the pO-DU.

If the servingCellMO IE in P O-DU to S O-DU SCell Information IE is included in the UE ADDITION REQUEST message, the sO-DU shall configure servingCellMO for the indicated SCell accordingly.

The maxMIMO-LayersDL-PerDU IE specifies the maximum number of DL MIMO layers that can be configured by the sO-DU across all the SCells for a given UE. If the maxMIMO-LayersDL IE in the P O-DU to S O-DU SCell Information is present, each SCell shall not configure the DL MIMO layers exceeding the value indicated by this IE. The cumulative number of configured DL MIMO layers across all SCells within the sO-DU shall not surpass the maxMIMO-LayersDL-PerDU IE value. In case of absence of maxMIMO-LayersDL IE in the P O-DU to S O-DU SCell Information, sO-DU can decide to configure DL MIMO layers in each of the SCells configured for the UE. The sO-DU shall report the total number of layers allocated for each SCell for the UE using the maxMIMO-LayersUsedDL IE in the S O-DU to P O-DU SCell Information while sending UE ADDITION RESPONSE message.

NOTE: The pO-DU is responsible for ensuring that the configuration provided to the sO-DU for maximum MIMO layers for downlink is in accordance with the network specified UE configuration (such as band combination).

sO-DU shall use ReportConfigType IE that contains the PUCCH allocation and CSI report slot configuration for the SCell, while forming the CSI-ReportConfig IE. CSI-ReportConfig IE shall be sent back to pO-DU which is finally sent to UE in RRC Reconfiguration message.

sO-DU shall respond back for each D2 UE ID with corresponding entry in UE Added List or UE Failed To Add List  IE in UE ADDITION RESPONSE message.

The sO-DU shall report to the pO-DU, in the UE ADDITION RESPONSE message, the result of all the successful UE addition in the following way:

- D2 UE ID shall be added in UE Added List IE identified by D2 UE ID and C-RNTI IEs.

- A list of SCells for the UE that is successfully added shall be included in SCell Added List IE.

- SCell configuration as defined in 3GPP TS38.331 [5] for each of the successfully added SCell shall be included in SCellConfig IE in S O-DU to P O-DU SCell Information.

- If periodic CSI reporting is configured on the SCell, the expected periodic CSI report size for each of the successful added SCell is included in PeriodicCsiReportSize IE in S O-DU to P O-DU SCell Information IE.

- A list of LCs for each SCell that is successfully added shall be included in LC Added List IE.

- A list of LCs which failed to be added shall be included in the LC Failed to Add List IE. Cause shall be added to convey the failure reason.

- SCell(s) that failed to be added in sO-DU, shall be included in the SCells Failed to Add List IE. Cause shall be added to convey the failure reason. D2-u procedure shall not take place for theese SCell(s).

If none of the SCell for a UE(s) is added by the sO-DU, the sO-DU shall reply with the UE ADDITION RESPONSE message listing the failed UE(s) in the UE Failed To Add List IE for each corresponding D2 UE ID. Cause IE shall indicate the cause of failure for UE addition procedure.

UE Modification Update

The purpose of the UE modification update procedure is to modify the established D2 UE Context(s) in sO-DU. As shown in figure 5.3.2.2-1, pO-DU may initiate this procedure to add a new SCell(s) if it is able to allocate PUCCH resources to receive the CSI information for new SCell or delete or modify configuration of an already added SCell(s) for the UE. Configuration update for multiple UEs, each one identified by D2 UE ID, may be performed in single UE MODIFICATION REQUEST message.

SCell O-DU

PCell O-DU

UE MODIFICATION REQUEST

UE MODIFICATION RESPONSE

Upon reception of the UE MODIFICATION REQUEST message, the sO-DU shall perform the modifications for each UE(s),and reports the outcome in the UE MODIFICATION RESPONSE message as shown in Figure 5.3.2.2-1.

Figure 5.3.2.2-1: UE Modification Update procedure

The sO-DU shall consider UE To Be Modified List IE as a list of candidate UE(s) identified by D2 UE ID IE that are requested for modification in the UE MODIFICATION REQUEST message. sO-DU shall store the received C-RNTI in D2 UE Context of each UE.

If the SCell To AddMod Item IE is contained in the UE MODIFICATION REQUEST message, the sO-DU shall consider SCell To Be AddMod Item IE as a list of candidates SCell(s) to be added or modified for the UE. If the servingCellMO IE is included in the UE MODIFICATION REQUEST message, the sO-DU shall use it as needed for the indicated SCell.

If the UE-CapabilityRAT-ContainerList IE is included in the UE MODIFICATION REQUEST message, the sO-DU shall take this information into account for UE specific configurations.

The maxMIMO-LayersDL-PerDU IE specifies the maximum number of DL MIMO layers that can be configured by the sO-DU across all the SCells for a given UE. If the maxMIMO-LayersDL IE in the P O-DU to S O-DU SCell Information is present, each SCell shall not configure the DL MIMO layers exceeding the value indicated by this IE. The cumulative number of configured DL MIMO layers across all SCells within the sO-DU shall not surpass the maxMIMO-LayersDL-PerDU IE value. In case of absence of maxMIMO-LayersDL IE in the P O-DU to S O-DU SCell Information, sO-DU can decide to configure DL MIMO layers in each of the SCells configured for a UE. The sO-DU shall report the total number of layers allocated for each SCell for the UE using the maxMIMO-LayersUsedDL IE in the S O-DU to P O-DU SCell Information while sending UE MODIFICATION RESPONSE message.

NOTE:	The pO-DU is responsible for ensuring that the configuration provided to the sO-DU for maximum MIMO layers for downlink is in accordance with the specified UE configuration (such as band combination).

sO-DU shall use ReportConfigType IE that contains the PUCCH allocation, CSI report slot configuration for the SCell, while forming the CSI-ReportConfig IE. CSI-ReportConfig IE shall be sent back to pO-DU which is finally sent to UE in the RRC Reconfiguration message.

If the DRX-Config IE is contained in the UE MODIFICATION REQUEST message, the sO-DU shall use the provided value from the pO-DU.

If the LC To Be AddMod Item IE is contained in the UE MODIFICATION REQUEST message, the sO-DU shall add or update the QoS config and NSSAI information for each of the LCID.

If the LC To Be Delete Item IE is contained in the UE MODIFICATION REQUEST message, the sO-DU shall release the LCs mentioned. No response shall be sent for these LCs.

If the SCells To Be Delete Item IE is contained in the UE MODIFICATION REQUEST message, the sO-DU shall release the SCell(s) for the UE. No Response shall be sent for these SCells. sO-DU shall not handle any of D2-u messages that are received for the released SCell(s).

The sO-DU shall report to the pO-DU, in the UE MODIFICATION RESPONSE message, the result of all the UEs that are requested for modification by the UE MODIFICATION REQUEST message in the following way:

- The UE(s) identified by D2 UE ID IE that is(are) successfully modified shall be included in UE Modified List IE.  Various IEs shall be filled as mentioned below:

  - SCell(s) for each UE that is successfully added or modified shall be included in SCell Updated List IE.

    - SCell configuration as defined in 3GPP TS 38.331 [5] for each of the successfully added SCell shall be included in SCellConfig IE.

    - If periodic CSI reporting is configured on the SCell, the expected periodic CSI report size for each of the successful added SCell shall be included in PeriodicCsiReportSize IE in S O-DU to P O-DU SCell Information IE.

  - SCell(s) that failed to be added or modified, shall be included in the SCells Failed to AddMod List IE. Cause IE shall mention the cause of failure. SCell(s) that failed shall not be used for any D2-c or D2-u message exchange between pO-DU and sO-DU until it is successfully added.

  - SCell(s) present in SCell To Be Deleted List IE shall not be included in the SCell Failed To AddMod List.

  - LC(s) that is(are) successfully added or modified shall be included in LC Updated List IE.

  - LCs which failed to be added or modified shall be included in the LC Failed to AddMod List IE. Cause IE shall mention the cause of failure.

  - LC(s) present in LC To Be Deleted List IE shall not be included in the LC Failed to AddMod List.

- sO-DU shall include the D2 UE ID and C-RNTI IE for the UE(s) in UE Failed To Mod List IE, for which none of the modification is accepted that was provided in the UE MODIFICATION REQUEST message.

UE Modification Indication

The purpose of the UE Modification Indication procedure is to notify the pO-DU that sO-DU is unable to handle certain resources (e.g., SCell(s), Logical Channel(s), or UE context) for a UE(s), and hence these resources shall not be used for any further D2-u communication until reconfigured again.

SCell O-DU

PCell O-DU

UE MODIFICATION INDICATION

As shown in Figure 5.3.2.3-1, the sO-DU initiates the procedure by sending UE MODIFICATION INDICATION message to pO-DU.

Figure 5.3.2.3-1: UE Modification Indication procedure

If UE To Be Updated List IE is contained in the message, the pO-DU shall consider this list as a candidate UE(s) identified by D2 UE ID IE, that are notified for modification in the UE MODIFICATION INDICATION message.

- If the SCell To Be Released Item IE is contained in the UE MODIFICATION INDICATION message, the pO-DU shall remove the SCell(s) from the D2 UE Context. pO-DU may initiate SCell Deactivation procedure towards UE.  Later if pO-DU decides to add the same SCell again, it may trigger UE Addition or Modification Procedure for adding the SCell. Cause IE shall indicate the reason of release.

- If the LCs To Be Released List IE is contained in the UE MODIFICATION INDICATION message, the pO-DU shall remove the LC information for the UE from D2 UE Context. These LCs shall not be considered while scheduling data on the sO-DU. Cause IE shall indicate the reason of release.

If UE To Be Released List IE is contained in the message, pO-DU shall remove the D2 UE context for the UE(s) identified by D2 UE ID IE. No further communication for the UE(s) shall take place with sO-DU until the UE is added again by the UE Addition procedure.

UE Reconfiguration Complete

The purpose of the UE Reconfiguration Complete procedure is to notify the sO-DU that the UE has successfully completed an RRC Reconfiguration procedure where SCells that were configured in sO-DU are updated.

As shown in Figure 5.3.2.4-1, pO-DU shall initiate the UE RECONFIGURATION COMPLETE message to sO-DU. UEReconfigList IE shall include the D2 UE ID and C-RNTI of UE(s) for which pO-DU has received RRC Reconfiguration complete from UE.

SCell O-DU

PCell O-DU

UE RECONFIGURATION COMPLETE

Figure 5.3.2.4-1: UE Reconfiguration Complete procedure

UE Deletion

The purpose of the UE Deletion procedure is to delete the D2 UE Context. This procedure erases any existing UE related information in sO-DU.

SCell O-DU

PCell O-DU

UE DELETION REQUEST

UE DELETION RESPONSE

As shown in Figure 5.3.2.5-1, the pO-DU initiates the procedure by sending UE DELETION REQUEST to sO-DU.

Figure 5.3.2.5-1: UE Deletion procedure

The sO-DU shall consider the UE(s) identified by D2 UE ID IE included in UE To Be Deleted List IE to delete the D2 UE context. sO-DU sends UE DELETION RESPONSE to pO-DU. Any ongoing D2-u procedure will be deleted for the UE and no new procedure will take place until UE is added again.

Elements of D2CP Communication

    - General

Subclauses 5.4.2 and 5.4.3 present the D2CP message and IE definitions in tabular format. The corresponding ASN.1

definition is presented in subclause 5.4.4. In case there is contradiction between the tabular format and the ASN.1

definition, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional IEs,

where the tabular format shall take precedence.

The messages have been defined in accordance to the guidelines specified in TR 25.921 [8].

When specifying IEs which are to be represented by bitstrings, if not otherwise specifically stated in the semantics

description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

- The first bit (leftmost bit) contains the most significant bit (MSB);

- The last bit (rightmost bit) contains the least significant bit (LSB);

- When importing bitstrings from other specifications, the first bit of the bitstring contains the first bit of the

concerned information;

The following attributes are used for the tabular description of the messages and information elements: Presence, Range

Criticality and Assigned Criticality. Their definition and use can be found in TS 38.413 [3].

Message Functional Definition and Content

Interface management functions

D2 SETUP REQUEST

This message is sent by the O-DU1 to transfer information associated to a D2-c connection.

Direction: O-DU1  O-DU2

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Served Cell List | M |  |  | List of SCells configured in the O-DU for inter O-DU CA | YES | reject |
| >Served Cell Item |  | 1 .. <maxInterDUSCells> |  |  |  |  |
| >>Served Cell Information | M |  |  | This IE contains cell configuration information of a cell in the O-DU as defined in 38.473 [6] section 9.3.1.10 | - | - |
| >>CA Type | M |  | 5.4.3.29 |  | YES | reject |
| >>Start CRNTI | O |  | 5.4.3.6 | This IE indicates the Start of C-RNTI that will be assigned to the UEs for which PCell is configured on this O-DU |  |  |
| >>End CRNTI | O |  | 5.4.3.6 | This IE indicates the end of C-RNTI that will be assigned to the UEs for which PCell is configured on this O-DU. |  |  |
| D2U Info List | M |  |  |  | YES | reject |
| >D2U Info Item |  | 1 .. < maxSupportedD2U > |  |  |  |  |
| >>D2U UDP Info List | M |  |  | List indicating the IP address and UDP port number for D2-u message exchange | YES | reject |
| >>>D2U UDP Info Item |  | 1 .. < maxSupportedD2U > |  |  |  |  |
| >>>>IP Address | M |  | BIT STRING (SIZE(1..160, ...)) | The IPv4 or IPv6 address of the O-DU1 that shall be used for D2-u communication with O-DU2.. |  |  |
| >>>>UDP port number | M |  | BIT STRING (SIZE(16)) | The UDP port number of O-DU1 that shall be used for D2-u communication with O-DU2. |  |  |
| >>D2U PV List | M |  |  |  | YES | reject |
| >>>D2U PV Item |  | 1 .. <maxSupportedPV> |  |  |  |  |
| >>>>Protocol Version | M |  | 5.4.3.13 |  |  |  |
| >>T_ClearTB | M |  | 5.4.3.31 |  | YES | reject |

D2 SETUP RESPONSE

This message is sent by the O-DU2 to transfer information associated to a D2-C connection.

Direction: O-DU2  O-DU1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Served Cell List | M |  |  | List of SCells configured in the O-DU for inter O-DU CA | YES | reject |
| >Served Cell Item |  | 1 .. <maxInterDUSCells> |  |  |  |  |
| >>Served Cell Information | M |  |  | This IE contains cell configuration information of a cell in the O-DU as defined in 38.473 [6] section 9.3.1.10 | - | - |
| >>CA Type | M |  | 5.4.3.29 |  | YES | reject |
| >>Start CRNTI | O |  | 5.4.3.6 | This IE indicates the Start of C-RNTI that will be assigned to the UEs for which PCell is configured on this O-DU |  |  |
| >>End CRNTI | O |  | 5.4.3.6 | This IE indicates the end of C-RNTI that will be assigned to the UEs for which PCell is configured on this O-DU. |  |  |
| D2U Info List | M |  |  |  | YES | reject |
| >D2U Info Item |  | 1 .. < maxSupportedD2U > |  |  |  |  |
| >>D2U UDP Info List | M |  |  | List indicating the IP address and UDP port number for D2-u message exchange | YES | reject |
| >>>D2U UDP Info Item |  | 1 .. < maxSupportedD2U > |  |  |  |  |
| >>>>IP Address | M |  | BIT STRING (SIZE(1..160, ...)) | The IPv4 or IPv6 address of the O-DU2 that shall be used for D2-U communication with O-DU1. |  |  |
| >>>>UDP port number | M |  | BIT STRING (SIZE(16)) | The UDP port number of O-DU2 that shall be used for D2-U communication with O-DU1. |  |  |
| >>D2U PV List | M |  |  |  | YES | reject |
| >>>D2U PV Item |  | 1 .. <maxSupportedPV> |  |  |  |  |
| >>>>Protocol Version | M |  | 5.4.3.13 |  |  |  |
| >>T_ClearTB | M |  | 5.4.3.31 |  | YES | reject |

D2 SETUP	FAILURE

This message is sent by the O-DU2 to indicate D2 Setup failure.

Direction: O-DU2  O-DU1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Cause | M |  | 5.4.3.3 |  | YES | ignore |
| Time to wait | O |  | 5.4.3.30 |  | YES | Ignore |

D2 CONFIGURATION UPDATE

This message is sent by the O-DU1 to transfer updated information for a D2-C interface connection.

Direction: O-DU1  O-DU2

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Served Cell To Add List |  | 0..1 |  | List of cells configured in the O-DU | YES | reject |
| >Served Cells To Add Item |  | 1.. <maxInterDUSCells > |  |  | EACH | reject |
| >>Served Cell Information | M |  |  | This IE contains cell configuration information of a cell in the O-DU as defined in 38.473 [6] section 9.3.1.10 | - | - |
| >>CA Type | M |  | 5.4.3.29 |  | YES | reject |
| >>Start CRNTI | O |  | 5.4.3.6 | Start of CRNTI used for this O-DU |  |  |
| >>End CRNTI | O |  | 5.4.3.6 | End of CRNTI used for this O-DU |  |  |
| Served Cell To Modify List |  | 0..1 |  | List of cells configured in the O-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. < <maxInterDUSCells> |  |  | EACH | reject |
| >>Old NR CGI | M |  | 5.4.3.7 |  |  |  |
| >>Served Cell Information | M |  |  | This IE contains cell configuration information of a cell in the O-DU as defined in 38.473 [6] section 9.3.1.10 |  |  |
| >>CA Type | M |  | 5.4.3.29 |  | YES | reject |
| >>Start CRNTI | O |  | 5.4.3.6 | Start of CRNTI used for this O-DU |  |  |
| >>End CRNTI | O |  | 5.4.3.6 | End of CRNTI used for this O-DU |  |  |
| Served Cell To Delete List |  | 0..1 |  | Complete list of deleted cells served by O-DU | YES | reject |
| >Served Cells To Delete Item |  | 1 .. <maxInterDUSCells> |  |  | EACH | reject |
| >>Old NR CGI | M |  | 5.4.3.7 |  |  |  |

D2 CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the O-DU2 to transfer information associated to a D2-c interface connection.

Direction: O-DU2  O-DU1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Updated Cell List |  | 0..1 |  | List of SCells configured in the O-DU for inter O-DU CA | YES | reject |
| >Updated Cells Item |  | 1.. <maxInterDUSCells> |  |  | EACH | reject |
| >>NR CGI | M |  | 5.4.3.7 |  | - | - |
| >>NR PCI | O | INTEGER (0..1007) |  | Physical Cell ID | YES | reject |
| >>Result | M |  | 5.4.3.8 | Indicates if configuration update for the cell is successful or failed | YES | reject |

D2 CONFIGURATION UPDATE FAILURE

This message is sent by the O-DU2 to indicate D2 Configuration Update failure.

Direction: O-DU2  O-DU1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | reject |
| Cause | M |  | 5.4.3.3 |  | YES | ignore |
| Time to wait | O |  | 5.4.3.30 |  | YES | ignore |

D2 RELEASE REQUEST

This message is sent by the O-DU1 to the O-DU2.

Direction: O-DU1  O-DU2

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | Reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | Reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | Reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | Reject |
| cause | M |  | 5.4.3.3 |  | YES | Ignore |

D2 RELEASE RESPONSE

This message is sent by the O-DU2 to the O-DU1.

Direction: O-DU2  O-DU1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | YES | Reject |
| Transaction ID | M |  | 5.4.3.2 |  | YES | Reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | YES | Reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | YES | Reject |

D2 ERROR INDICATION

This message is sent by both the O-DUs and is used to indicate that some error has been detected in the node.

Direction: O-DU1  O-DU2

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 |  | Reject |
| Target O-DU ID | M |  | 5.4.3.4 |  | Reject |
| Cause | M |  |  | Indicates the cause that triggered the Error Indication | Reject |

UE management functions

UE ADDITION REQUEST

This message is sent from pO-DU to add the SCell(s) from sO-DU for the listed UEs.

Direction: pO-DU  sO-DU.

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| UE To Be Added List |  | 1 |  | List of UEs for which request is received | Reject |
| > UE To Be Added Item |  | 1 .. < maxInterDUUEs> |  |  | ignore |
| >> C-RNTI | M |  | 5.4.3.6 |  |  |
| >> D2 UE ID | M |  | 5.4.3.5 |  |  |
| >> maxMIMO-LayersDL-PerDU | M |  | INTEGER (1..32) | indicates the maximum number of DL MIMO layers that can be configured for a UE in the sO-DU. If there are multiple SCells configured for the UE in the sO-DU, total layers across all the SCells shall not exceed this value. | ignore |
| >> UE-CapabilityRATContainerList | M |  | OCTET STRING | This IE consists of the UE-CapabilityRATContainerList, as defined in TS 38.331 [5]. | Reject |
| >> SCells To Be Added List |  | 1 |  | List of SCells that has to be added. | Reject |
| >>> SCell To Be Added Item |  | 1 .. <maxInterDUSCells> |  |  | ignore |
| >>>> P O-DU to S O-DU SCell Information | M |  | 5.4.3.15 |  |  |
| >>MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in 3GPP TS 38.331 [5]. | ignore |
| >>DRX-Config | O |  | OCTET STRING | DRX-Config, as defined in 3GPP TS 38.331 [5]. | ignore |
| >>LC To Be Added List |  | 1 |  | List of LCs which can be added for SCell | reject |
| >>>LC To Be Added Item |  | 1 .. <maxLCs> |  |  | ignore |
| >>>>LCID | M |  | 5.4.3.10 | LC Identifier as defined in 3GPP TS 38.473 [6] clause 9.3.1.35 |  |
| >>>> QoS Information | M |  | 5.4.3.22 |  |  |
| >>>>S-NSSAI | M |  | 5.3.4.21 | Corresponds to S-NSSAI as defined in 3GPP TS 38.473 [6] clause 9.3.1.38 |  |

UE ADDITION RESPONSE

This message is sent from sO-DU to pO-DU as a response to UE ADDITION REQUEST message. This message is sent for both success and failure scenario.

Direction: sO-DU  pO-DU

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 3GPP TS 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 3GPP TS 38.473 [6] clause 9.3.1.9. | Reject |
| UE Added List |  | 0..1 |  | List of UEs that are added successfully | ignore |
| > UE Added Item |  | 1 .. < maxInterDUUEs> |  |  | ignore |
| >> C-RNTI | M |  | 5.4.3.6 |  |  |
| >> D2 UE ID | M |  | 5.4.3.5 |  |  |
| >> SCells Added List |  | 0..1 |  | List of SCells that is successfully added. | Reject |
| >>> SCell Added Item |  | 1 .. <maxInterDUSCells> |  |  | ignore |
| >>>> S O-DU to P O-DU SCell Information | M |  | 5.4.3.17 |  |  |
| >>LC Added List |  | LCList | 5.4.3.11 | List of LCs which are added successfully | reject |
| >>LC Failed To Add List |  | LC Release List | 5.4.3.12 |  | reject |
| >>SCell Failed To Add List |  | SCell Release List | 5.4.3.16 |  | reject |
| UE Failed To Add List |  | UE Release List | 5.4.3.19 |  | ignore |

UE DELETION REQUEST

pO-DU initiates the message to delete the D2 UE context from sO-DU for the listed UEs.

Direction:  pO-DU  sO-DU

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| UE To Be Deleted List | M | UE List | 5.4.3.18 | List of UEs for which request is received | Reject |

UE DELETION RESPONSE

sO-DU triggers UE DELETION RESPONSE message to respond back to UE DELETION REQUEST.

Direction:  sO-DU  pO-DU

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |

UE MODIFICATION REQUEST

This message is sent from pO-DU to add a new SCell or modify or delete the existing SCell from sO-DU.

Direction: pO-DU  sO-DU.

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| UE To Be Modified List |  | 1 |  | List of UEs for which request is received | Reject |
| > UE To Be Modified Item |  | 1 .. < maxInterDUUEs> |  |  | ignore |
| >> C-RNTI | M |  | 5.4.3.6 |  |  |
| >> D2 UE ID | M |  | 5.4.3.5 |  |  |
| >> maxMIMO-LayersDL-PerDU | M |  | 5.4.3.28 | This IE indicates the maxMIMO-Layers that can be supported for a UE in the sO-DU. If there are multiple SCells configured for the UE, total layers across all the SCells shall not exceed this value | reject |  |
| >> UE-CapabilityRATContainerList | O |  | OCTET STRING | Same as defined in 38.331 [5] | ignore |
| >> SCells To AddMod List |  | 0..1 |  | List of SCells that has to be added. | Reject |
| >>> SCell To AddMod Item |  | 1 .. <maxInterDUSCells> |  |  | ignore |
| >>>> P O-DU to S O-DU SCell Information | M |  | 5.4.3.14 |  |  |
| >>MeasGapConfig | O |  | OCTET STRING | Corresponds to MeasGapConfig IE as defined in 38.331 [5] | ignore |
| >>DRX-Config | O |  | OCTET STRING | Corresponds to DRX-Config IE as defined in 38.331 [5] | ignore |
| >>LC To Be AddMod List |  | 0..1 |  | List of LCs which can be added for SCell | reject |
| >>>LC To Be AddMod Item |  | 1 .. <maxLCs> |  |  | ignore |
| >>>>LCID | M |  | 5.4.3.10 | LC Identifier |  |
| >>>> QoS Information | M |  | 5.4.3.21 |  |  |
| >>>>S-NSSAI | M |  | 5.3.4.20 |  |  |
| >>LC To Be Deleted List |  | LC List | 5.4.3.11 | List of LCs which need to be deleted for UE | reject |
| >>SCell To Be Deleted List |  | SCell List | 5.4.3.15 | List of SCells which need to be released. | reject |

UE MODIFICATION RESPONSE

This message is sent from sO-DU to pO-DU as a response to UE MODIFICATION REQUEST message. This message contains all the UEs, SCells and LCs that are successfully modified. List of failed UEs, SCells , LCs are also included in the message

Direction: sO-DU  pO-DU.

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| UE Modified List |  | 0..1 |  | List of modified UEs | ignore |
| > UE Modified Item |  | 1 .. < maxInterDUUEs> |  |  |  |
| >> C-RNTI | M |  | 5.4.3.6 |  |  |
| >> D2 UE ID | M |  | 5.4.3.5 |  |  |
| >> SCells Updated List |  | 0..1 |  | List of modified SCells. | Reject |
| >>> SCell Updated Item |  | 1 .. <maxInterDUSCells> |  |  | ignore |
| >>>> S O-DU to P O-DU SCell Information | M |  | 5.4.3.17 |  |  |
| >>LC Updated List |  | 0..1 | LCList 5.4.3.11 | List of modified LCs. | reject |
| >>LC Failed To AddMod List |  | 0..1 | LC Release List 5.4.3.12 | List of LCs which failed to be modified. | reject |
| >>SCell Failed To AddMod List |  | 0..1 | SCell Release List 5.4.3.16 | List of SCells which failed to be modified | reject |
| UE Failed To Mod List |  | 0..1 | UE Release List 5.4.3.19 | List of UE failed to be Modified. | ignore |

UE RECONFIGURATION COMPLETE

pO-DU initiates this message towards SCell O_DU to inform the reception of RRC Reconfiguration Complete message from UE for SCell addition.

Direction: pO-DU  sO-DU

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| UE Reconfig List | M |  | UE Reconfig List 5.4.3.27 | List of UEs for which reconfiguration complete message is received | Reject |

UE MODIFICATION INDICATION

sO-DU initiates this message to inform pO-DU about release of some of the SCells or LCs or UEs between a pair of O-DUs.

Direction: sO-DU  pO-DU

| IE/Group Name | Presence | Range | IE Type and reference | Semantics Description | Assigned Criticality |
| --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 5.4.3.1 |  | reject |
| Transaction ID | M |  | 5.4.3.2 |  | Reject |
| Source O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| Target O-DU ID | M |  | 5.4.3.4 | This is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9. | Reject |
| UE To Be Updated List |  | 0..1 |  | List of UEs for which request is received | Reject |
| > UE To Be Updated Item |  | 1 .. < maxInterDUUEs> |  |  | ignore |
| >> C-RNTI | M |  | 5.4.3.6 |  |  |
| >> D2 UE ID | M |  | 5.4.3.5 |  |  |
| >>SCells To Be Released List |  | 0..1 | SCell Release List 5.4.3.16 | List of SCells which are released. | ignore |
| >>LC To Be Released List |  | 0..1 | LC Release List 5.4.3.12 | List of LCs which are released | ignore |
| UE To Be Released List |  | 0..1 | UE Release List 5.4.3.19 | List of UEs that are released | ignore |

Information Element Definitions

Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | INTEGER (0..255) |  |
| >Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, ...) |  |

Transaction ID

The Transaction ID IE uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transaction ID | M |  | INTEGER (0..255, …) |  |

Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the D2 protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Cause | M |  | ENUMERATED (Unspecified, Scell Down, UE Not Allowed, Cell Overload … etc) |  |

| Protocol cause | Meaning |
| --- | --- |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated “reject”. |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated “ignore and notify”. |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |
| SCell Down | SCell is down in DU |
| UE Not Allowed | Admission control failed in DU for some reason |
| Cell Overload | Cell is overloaded hence can not handle anymore UE. |
| CQI Failure | CQI is not received for configured duration by sO-DU hence releasing the SCell. |
| HARQ Failure | HARQ Feedback is not received for the configured duration |
| PV Not supported | Protocol version is not supported |
| UDP Port Invalid | UDP port information exchanged in UDP info is not supported. |
| Success |  |

O-DU ID

The O-DU ID IE uniquely identifies the O-DU at least within a O-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| O-DU ID | M |  | INTEGER (0 .. 236-1) | The O-DU ID is independently configured from cell identifiers, i.e. no connection between O-DU ID and cell identifiers. It is same as gNB-DU-ID as defined in 38.473 [6] clause 9.3.1.9 |

D2 UE ID

The D2 UE  ID IE uniquely identifies the UE association over the D2 interface between a pair of O-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| D2 UE ID | M |  | INTEGER (0 .. 216 -1) |  |

C-RNTI

The C-RNTI IE is the UE identifier used between O-DU and UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| C-RNTI | M |  | INTEGER (0 .. 232 -1) | As defined in 38.473 [6] clause 9.3.1.32 |

NR CGI

The NR Cell Global Identifier (NR CGI IE) is used to globally identify a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 5.4.3.9 | Same as defined in 3GPP TS 38.473 [6] clause 9.3.1.14 |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  |

Result

This IE indicates the outcome of a procedure whether it is success or failure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Result | M |  | ENUMERATED (Success, Failure, ...) |  |

PLMN Identity

This information element indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE(3)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n- 1 - bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |

LCID

This IE uniquely identifies a LC for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LCID | M |  | INTEGER (1..32) | Corresponds to the LogicalChannelIdentity defined in 3GPP TS 38.331 [5]. |

LCList

This IE indicates the list of LCs

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LC List Item IEs |  | 1 .. <maxLCs> |  |  |
| LCID | M |  | 5.4.3.10 | Corresponds to the LogicalChannelIdentity defined in 3GPP TS 38.331 [5]. |

LC Release List

This IE indicates the list of LCs that are failed to be added or modified.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LC List Item IEs |  | 1 .. <maxLCs> |  |  |
| >LCID | M |  | 5.4.3.10 | Corresponds to the LogicalChannelIdentity defined in 3GPP TS 38.331 [5]. |
| >Cause | M |  | 5.4.3.3 |  |

Protocol Version

This IE uniquely identifies the D2-u protocol version value.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Protocol Version | M |  | INTEGER (0..16) | D2 Protocol version |

P O-DU to S O-DU SCell Information

This IE contains the SCell information passed from pO-DU to sO-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCell ID | M | INTEGER (0 .. 236-1) | NR CGI 5.4.3.7 | The NR Cell Global Identifier (NR CGI) is used to globally identify a SCell. It is same as defined in 38.473 [6] clause 9.3.1.12. This shall contain the same value as received from O-CU. |
| SCellIndex | M | INTEGER (1..31) |  | Index for the SCell as received from O-CU. This is same as defined in 38.331 [5] clause 6.3.2. |
| servingCellMO | O | INTEGER (1..64) |  | as specified in 38.331 [5] clause 6.3.2 |
| maxMIMO-LayersDL | O |  | 5.4.3.28 |  |
| reportConfigType | M | OCTET STRING |  | This IE shall be encoded as specified 3GPP TS 38.331 [5] clause 6.3.2. In the present document only Periodic CSI reporting is supported. |

SCell List

This IE indicates the list of SCells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCell List Item IEs |  | 1 .. < maxInterDUSCells > |  |  |
| >SCell ID | M | INTEGER (0 .. 236-1) | NR CGI 5.4.3.7 | The NR Cell Global Identifier (NR CGI) is used to globally identify a SCell. It is same as defined in 38.473[6] clause 9.3.1.12. This shall contain the same value as received from O-CU. |
| >SCellIndex | M | INTEGER (1..31) |  | Index for the SCell as received from O-CU. This is same as defined in 38.331 [5] clause 6.3.2. |

SCell Release List

This IE indicates the list of SCells that needs to be released.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCell List Item IEs |  | 1 .. < maxInterDUSCells > |  |  |
| >SCell ID | M | INTEGER (0 .. 236-1) | NR CGI 5.4.3.7 | The NR Cell Global Identifier (NR CGI) is used to globally identify a SCell. It is same as defined in 38.473[6] clause 9.3.1.12. This shall contain the same value as received from O-CU. |
| >SCellIndex | M | INTEGER (1..31) |  | Index for the SCell as received from O-CU. This is same as defined in 38.331 [5] clause 6.3.2. |
| >Cause | M |  | 5.4.3.3 |  |

S O-DU to P O-DU SCell Information

This IE contains the SCell configuration information passed from sO-DU to pO-DU. This information will be sent to UE by pO-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCell ID | M | INTEGER (0 .. 236-1) | NR CGI 5.4.3.7 | The NR Cell Global Identifier (NR CGI) is used to globally identify a SCell. It is same as defined in 38.473[6] clause 9.3.1.12. This shall contain the same value as received from O-CU. |
| SCellIndex | M | INTEGER (1..31) |  | Index for the SCell as received from O-CU. This is same as defined in 38.331 [5] clause 6.3.2. |
| SCellConfig | M | OCTET STRING |  | Corresponds to SCellConfig, as defined in 3GPP TS 38.331 [5] clause 6.3.2. |
| maxMIMO-LayersUsedDL | M | 5.4.3.28 |  | Indicates the maxMIMO-layers used for the UE in the SCell |
| CSI-ReportConfig | M | OCTET STRING |  | Same as CSI-ReportConfig as defined in 38.331 |
| PeriodicCsiReportSize | O | INTEGER (1… 3000,...) |  | The unit is: bits This IE shall be present if CSI-Report Type is Periodic. |

UE List

This IE indicates the list of UEs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE List Item IEs |  | 1 .. < maxInterDUUEs > |  |  |
| >C-RNTI | M |  | C-RNTI 5.4.3.6 | Corresponds to the C-RNTI defined in 3GPP TS 38.473 [6] clause 9.3.1.32. |
| >D2 UE ID | M |  | 5.4.3.5 |  |

UE Release List

This IE indicates the list of UEs that needs to be released or that are released.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE List Item IEs |  | 1 .. < maxInterDUUEs > |  |  |
| >C-RNTI | M |  | C-RNTI 5.4.3.6 | Corresponds to the C-RNTI defined in 3GPP TS 38.473 [6] clause 9.3.1.32. |
| >D2 UE ID | M |  | 5.4.3.5 |  |
| >Cause | M |  | 5.4.3.3 |  |

S-NSSAI

This IE indicates the S-NSSAI as defined in TS 23.003 [15].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SST | M | OCTET STRING (SIZE(1)) |  |  |
| SD | O | OCTET STRING (SIZE(3)) |  |  |

QoS Information

This IE uniquely defines the QoS parameter for each LC identified by LCID for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| nr5QI | M |  | INTEGER (0..255) | This IE contains the standardized 5QI as specified in TS 23.501 [16]. |
| NG-RAN Allocation and Retention Priority | M |  | 5.3.4.28 | This IE is same as defined in 3GPP TS 38.473 [6] clause 9.3.1.48 |
| qoSPriorityLevel | M |  | INTEGER (1..127) | For details see 3GPP TS 23.501 [16] |
| packetDelayBudget | M |  | 5.4.3.24 | For details see 3GPP TS 23.501 [16] |
| packetErrorRate | M |  | 5.4.3.23 | For details see 3GPP TS 23.501 [16] |
| delayCritical | M |  | ENUMERATED {delay-critical, non-delay-critical} | For details see 3GPP TS 23.501 [16] |
| GBR QoS Flow Information | O |  | 5.3.4.26 | This is same as defined in 3GPP TS 38.473 [6] clause 9.3.1.46 |

Packet Error Rate

This IE indicates the packet error rate for a LCID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Scaler | M |  | INTEGER (0..9, ...) | The packet error rate is expressed as Scalar x 10-k where k is the Exponent. |
| Exponent | M |  | INTEGER (0..9, ...) |  |

Packet Delay Budget

This IE indicates the packet delay budget for a LCID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Delay Budget | M |  | INTEGER (0..1023, …) | Upper bound value for the delay that a packet may experience expressed in unit of 0.5ms. |

NG-RAN Allocation and Retention Priority

This IE specifies the relative importance of a LCID for allocation and retention of NG-RAN resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (0..15) | Desc.: This IE defines the relative importance of a resource request (see TS 23.501 [16]). Usage: Values are ordered in decreasing order of priority, i.e., with 1 as the highest priority and 15 as the lowest priority. Further usage is defined in TS 23.501 [16]. |
| Pre-emption Capability | M |  | ENUMERATED (shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other QoS flows (see TS 23.501 [16]). Usage: The QoS flow shall not pre-empt other QoS flows or, the QoS flow may pre-empt other QoS flows. Note: The Pre-emption Capability indicator applies to the allocation of resources for a QoS flow and as such it provides the trigger to the pre-emption procedures/processes of the NG-RAN node. |
| Pre-emption Vulnerability | M |  | ENUMERATED (not preemptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the QoS flow to pre-emption of other QoS flows (see TS 23.501 [16]). Usage: The QoS flow shall not be pre-empted by other QoS flows or the QoS flow may be pre-empted by other QoS flows. Note: The Pre-emption Vulnerability indicator applies for the entire duration of the QoS flow, unless modified and as such indicates whether the QoS flow is a target of the preemption procedures/processes of the NG-RAN node. |

GBR QoS Flow Information

This IE indicates QoS parameters for a LCID that is configured for GBR QoS flow or GBR bearer for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | BitRate 5.3.4.27 | Maximum Bit Rate in DL. Details in TS 23.501 [16]. |
| Maximum Flow Bit Rate Uplink | M |  | BitRate 5.3.4.27 | Maximum Bit Rate in UL. Details in TS 23.501 [16]. |
| Guaranteed Flow Bit Rate Downlink | M |  | BitRate 5.3.4.27 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [16]. |
| Guaranteed Flow Bit Rate Uplink | M |  | BitRate 5.3.4.27 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [16]. |
| Maximum Packet Loss Rate Downlink | O |  | INTEGER(0..1000) | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Details in TS 23.501 [16]. |
| Maximum Packet Loss Rate Uplink | O |  | INTEGER(0..1000) | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Details in TS 23.501 [16]. |

BitRate

This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL within a period of time, divided

by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a LCID configured for a GBR QoS flow, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0… 4,000,000,000,000,...) | The unit is: bit/s |

UE Reconfig List

This IE indicates the list of UEs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE List Item IEs |  | 1 .. < maxInterDUUEs > |  |  |
| >C-RNTI | M |  | C-RNTI 5.4.3.6 | Corresponds to the C-RNTI defined in 3GPP TS 38.473 [6] clause 9.3.1.32. |
| >D2 UE ID | M |  | 5.4.3.5 |  |
| >SCell List | C | SCell List | 5.3.4.16 | This IE shall be present if UE List is encoded in UE RECONFIGURATION COMPLETE message |

maxMIMO-Layers

This IE indicates the maximum DL MIMO layers.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| maxMIMO-Layers | M |  | INTEGER ( 1…16) | This is same as maxMIMO-Layers as defined in 3GPP TS 38.331 [5] |

CA Type

This IE indicates CA type supported by the cell. In the present version of specification, only DL CA is supported.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CA Type | M |  | ENUMERATED (DL, …) |  |

Time to wait

This IE defines the minimum wait time for O-DU before retrying the message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time to wait | O |  | ENUMERATED (1s, 2s, 5s, 10s, 20s, 60s, 120s, …) | Indicates the time value in seconds. |

T_ClearTB

This IE is used in D2-U procedure for retransmission of the data. Once data is transmitted on SCell, T_ClearTB timer shall be started. The timer is stopped either on successful HARQ feedback, or on reception of DL SCHEDULING REQUEST message containing the retransmission request for same HARQ PID for the same UE in same SCell. On expiry of T_ClearTB, sO-DU shall Clear the HARQ process and make the HARQ PID available for next transmission with toggled NDI bit.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| T_ClearTB | M |  | ENUMERATED (ms1, ms2, ms5, ms10, ms20, ms60, …) | Indicates the time value in ms. |

Message and Information Element Abstract Syntax (with ASN.1)

General

D2CP ASN.1 definition conforms to ITU-T Recommendation X.691 [12], ITU-T Recommendation X.680 [13] and ITUT Recommendation X.681 [14].

The ASN.1 definition specifies the structure and content of D2-C messages. D2CP messages can contain any IEs specified in the object set definitions for that message without the order or number of occurrences being restricted by ASN.1. However, for this version of the standard, a sending entity shall construct aD2-C message according to the PDU definitions module and with the following additional rules:

- IEs shall be ordered (in an IE container) in the order they appear in object set definitions.

- Object set definitions specify how many times IEs may appear. An IE shall appear exactly once if the presence

field in an object has value "mandatory". An IE may appear at most once if the presence field in an object has

value "optional" or "conditional". If in a tabular format there is multiplicity specified for an IE (i.e., an IE list)

then in the corresponding ASN.1 definition the list definition is separated into two parts. The first part defines an

IE container list where the list elements reside. The second part defines list elements. The IE container list

appears as an IE of its own. For this version of the standard an IE container list may contain only one kind of list

elements.

NOTE: In the above "IE" means an IE in the object set with an explicit ID. If one IE needs to appear more than

once in one object set, then the different occurrences will have different IE IDs.

If a D2CP message that is not constructed as defined above is received, this shall be considered as Abstract Syntax

Error, and the message shall be handled as defined for Abstract Syntax Error.

Usage of private message mechanism for non-standard use

The private message mechanism for non-standard use is not defined for D2CP messages.

Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

-- **************************************************************

D2CP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) d2cp (4) version1 (1) d2cp-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM D2CP-CommonDataTypes

D2ErrorIndication,

D2SetupRequest,

D2SetupResponse,

D2SetupFailure,

D2ConfigurationUpdate,

D2ConfigurationUpdateAcknowledge,

D2ConfigurationUpdateFailure,

D2ReleaseRequest,

D2ReleaseResponse,

UEAdditionRequest,

UEAdditionResponse,

UEDeletionRequest,

UEDeletionResponse,

UEModificationRequest,

UEModificationResponse,

UEModificationIndication,

UEReconfigurationComplete

FROM D2CP-PDU-Contents

id-d2errorIndication,

id-d2Setup,

id-d2ConfigurationUpdate,

id-d2Release,

id-uEAddition,

id-uEDeletion,

id-uEModificationUpdate,

id-uEModificationIndication,

id-uEReconfigurationComplete

FROM D2CP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

D2CP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

D2CP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	D2CP-ELEMENTARY-PROCEDURE.&procedureCode		({D2CP-ELEMENTARY-PROCEDURES}),

criticality		D2CP-ELEMENTARY-PROCEDURE.&criticality			({D2CP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			D2CP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({D2CP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	D2CP-ELEMENTARY-PROCEDURE.&procedureCode			({D2CP-ELEMENTARY-PROCEDURES}),

criticality		D2CP-ELEMENTARY-PROCEDURE.&criticality			({D2CP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			D2CP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome		({D2CP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	D2CP-ELEMENTARY-PROCEDURE.&procedureCode			({D2CP-ELEMENTARY-PROCEDURES}),

criticality		D2CP-ELEMENTARY-PROCEDURE.&criticality		 	({D2CP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			D2CP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome 	({D2CP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

D2CP-ELEMENTARY-PROCEDURES D2CP-ELEMENTARY-PROCEDURE ::= {

D2CP-ELEMENTARY-PROCEDURES-CLASS-1 |

D2CP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

D2CP-ELEMENTARY-PROCEDURES-CLASS-1 D2CP-ELEMENTARY-PROCEDURE ::= {

d2Setup					|

d2ConfigurationUpdate 	|

d2Release				|

uEAddition				|

uEDeletion				|

uEModificationUpdate,

...

}

D2CP-ELEMENTARY-PROCEDURES-CLASS-2 D2CP-ELEMENTARY-PROCEDURE ::= {

d2errorIndication			|

uEModificationIndication	|

uEReconfigurationComplete,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

d2errorIndication D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		D2ErrorIndication

PROCEDURE CODE			id-d2errorIndication

CRITICALITY				ignore

}

d2Setup D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		D2SetupRequest

SUCCESSFUL OUTCOME		D2SetupResponse

UNSUCCESSFUL OUTCOME	D2SetupFailure

PROCEDURE CODE			id-d2Setup

CRITICALITY				reject

}

d2ConfigurationUpdate D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		D2ConfigurationUpdate

SUCCESSFUL OUTCOME		D2ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	D2ConfigurationUpdateFailure

PROCEDURE CODE			id-d2ConfigurationUpdate

CRITICALITY				reject

}

d2Release D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		D2ReleaseRequest

SUCCESSFUL OUTCOME		D2ReleaseResponse

PROCEDURE CODE			id-d2Release

CRITICALITY				reject

}

uEAddition D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEAdditionRequest

SUCCESSFUL OUTCOME		UEAdditionResponse

PROCEDURE CODE			id-uEAddition

CRITICALITY				reject

}

uEDeletion D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEDeletionRequest

SUCCESSFUL OUTCOME		UEDeletionResponse

PROCEDURE CODE			id-uEDeletion

CRITICALITY				reject

}

uEModificationUpdate D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEModificationRequest

SUCCESSFUL OUTCOME		UEModificationResponse

PROCEDURE CODE			id-uEModificationUpdate

CRITICALITY				reject

}

uEModificationIndication D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEModificationIndication

PROCEDURE CODE			id-uEModificationIndication

CRITICALITY				reject

}

uEReconfigurationComplete D2CP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEReconfigurationComplete

PROCEDURE CODE			id-uEReconfigurationComplete

CRITICALITY				ignore

}

END

-- ASN1STOP

PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for D2CP.

-- **************************************************************

D2CP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) d2cp (4) version1 (1) d2cp-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

D2U-Info-Item,

O-DU-ID,

SCell-Release-Item,

Served-Cell-Item,

ServedCellToMod-Item,

ServedCellToRel-Item,

TimeToWait,

TransactionID,

UE-Added-Item,

UE-Item,

UE-List,

UE-Modified-Item,

UE-Reconfig-List,

UE-Release-List,

UE-Release-Item,

UE-To-Be-Added-Item,

UE-ToBeModified-Item,

UE-ToBeUpdated-Item,

UpdatedCell-Item

FROM D2CP-IEs

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

D2CP-PROTOCOL-EXTENSION,

D2CP-PROTOCOL-IES,

D2CP-PROTOCOL-IES-PAIR

FROM D2CP-Containers

id-Cause,

id-D2U-Info-List,

id-D2U-Info-Item,

id-TimeToWait,

id-TransactionID,

id-Served-Cell-List,

id-Served-Cell-Item,

id-ServedCellToAdd-List,

id-ServedCellToMod-List,

id-ServedCellToMod-Item,

id-ServedCellToRel-List,

id-ServedCellToRel-Item,

id-Source-O-DU-ID,

id-Target-O-DU-ID,

id-UE-Added-Item,

id-UE-Added-List,

id-UE-Reconfig-List,

id-UE-Failed-To-Add-List,

id-UE-To-Be-Added-Item,

id-UE-To-Be-Added-List,

id-UE-FailedToAddMod-List,

id-UE-List,

id-UE-Reconfig-List,

id-UE-Item,

id-UE-Modified-List,

id-UE-Modified-Item,

id-UE-ToBeDeleted-List,

id-UE-ToBeReleased-List,

id-UE-ToBeModified-List,

id-UE-ToBeModified-Item,

id-UE-ToBeUpdated-List,

id-UE-ToBeUpdated-Item,

id-UpdatedCell-List,

id-UpdatedCell-Item,

maxInterDUUEs,

maxInterDUSCells,

maxLCs,

maxSupportedD2U,

maxSupportedPV

FROM D2CP-Constants;

-- **************************************************************

--

-- ERROR INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Error Indication

--

-- **************************************************************

D2ErrorIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container {{D2ErrorIndicationIEs}},

...

}

D2ErrorIndicationIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory}|

{ ID id-Source-O-DU-ID CRITICALITY ignore TYPE O-DU-ID PRESENCE optional }|

{ ID id-Target-O-DU-ID CRITICALITY ignore TYPE O-DU-ID PRESENCE optional }|

{ ID id-Cause CRITICALITY ignore TYPE Cause PRESENCE optional },

...

}

-- **************************************************************

--

-- D2 Setup ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- D2 SETUP REQUEST

--

-- **************************************************************

D2SetupRequest  ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2SetupRequestIEs} },

...

}

D2SetupRequestIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Served-Cell-List CRITICALITY reject TYPE Served-Cell-List PRESENCE mandatory }|

{ ID id-D2U-Info-List CRITICALITY reject TYPE D2U-Info-List PRESENCE mandatory },

...

}

Served-Cell-List ::= SEQUENCE (SIZE(1.. maxInterDUSCells)) OF ProtocolIE-SingleContainer { { Served-Cell-ItemIEs} }

Served-Cell-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-Served-Cell-Item CRITICALITY reject TYPE Served-Cell-Item PRESENCE mandatory },

...

}

D2U-Info-List ::= SEQUENCE (SIZE(1.. maxSupportedD2U)) OF ProtocolIE-SingleContainer { { D2U-Info-ItemIEs} }

D2U-Info-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-D2U-Info-Item CRITICALITY reject TYPE D2U-Info-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

-- D2 SETUP RESPONSE

--

-- **************************************************************

D2SetupResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2SetupResponseIEs} },

...

}

D2SetupResponseIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Served-Cell-List CRITICALITY reject TYPE Served-Cell-List PRESENCE optional }|

{ ID id-D2U-Info-List CRITICALITY reject TYPE D2U-Info-List PRESENCE mandatory },

...

}

-- **************************************************************

--

-- D2 SETUP FAILURE

--

-- **************************************************************

D2SetupFailure ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2SetupFailureIEs} },

...

}

D2SetupFailureIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Cause CRITICALITY ignore TYPE Cause PRESENCE mandatory }|

{ ID id-TimeToWait CRITICALITY ignore TYPE TimeToWait PRESENCE optional },

...

}

-- **************************************************************

--

-- D2 Configuration Update ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- D2 CONFIGURATION UPDATE

--

-- **************************************************************

D2ConfigurationUpdate  ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2ConfigurationUpdateIEs} },

...

}

D2ConfigurationUpdateIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-ServedCellToAdd-List CRITICALITY reject TYPE Served-Cell-List PRESENCE optional }|

{ ID id-ServedCellToMod-List CRITICALITY reject TYPE ServedCellToMod-List PRESENCE optional }|

{ ID id-ServedCellToRel-List CRITICALITY reject TYPE ServedCellToRel-List PRESENCE optional },

...

}

ServedCellToMod-List ::= SEQUENCE (SIZE(1.. maxInterDUSCells)) OF ProtocolIE-SingleContainer { { ServedCellToMod-ItemIEs} }

ServedCellToMod-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-ServedCellToMod-Item CRITICALITY reject TYPE ServedCellToMod-Item PRESENCE mandatory },

...

}

ServedCellToRel-List ::= SEQUENCE (SIZE(1.. maxInterDUSCells)) OF ProtocolIE-SingleContainer { { ServedCellToRel-ItemIEs} }

ServedCellToRel-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-ServedCellToRel-Item CRITICALITY reject TYPE ServedCellToRel-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

-- D2 CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

D2ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2ConfigurationUpdateAcknowledgeIEs} },

...

}

D2ConfigurationUpdateAcknowledgeIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UpdatedCell-List CRITICALITY reject TYPE UpdatedCell-List PRESENCE optional },

...

}

UpdatedCell-List ::= SEQUENCE (SIZE(1.. maxInterDUSCells)) OF ProtocolIE-SingleContainer { { UpdatedCell-ItemIEs} }

UpdatedCell-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-UpdatedCell-Item CRITICALITY reject TYPE UpdatedCell-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

-- D2 CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

D2ConfigurationUpdateFailure	::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2ConfigurationUpdateFailureIEs} },

...

}

D2ConfigurationUpdateFailureIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Cause CRITICALITY ignore TYPE Cause PRESENCE mandatory }|

{ ID id-TimeToWait CRITICALITY ignore TYPE TimeToWait PRESENCE optional },

...

}

-- **************************************************************

--

-- D2 Release ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- D2 RELEASE REQUEST

--

-- **************************************************************

D2ReleaseRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2ReleaseRequestIEs} },

...

}

D2ReleaseRequestIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Cause CRITICALITY ignore TYPE Cause PRESENCE mandatory },

...

}

-- **************************************************************

--

-- D2 RELEASE RESPONSE

--

-- **************************************************************

D2ReleaseResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { D2ReleaseResponseIEs} },

...

}

D2ReleaseResponseIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory },

...

}

-- **************************************************************

--

-- UE Addition ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE ADDITION REQUEST

--

-- **************************************************************

UEAdditionRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { UEAdditionRequestIEs} },

...

}

UEAdditionRequestIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UE-To-Be-Added-List CRITICALITY reject TYPE UE-To-Be-Added-List PRESENCE mandatory },

...

}

UE-To-Be-Added-List ::= SEQUENCE (SIZE(1.. maxInterDUUEs)) OF ProtocolIE-SingleContainer { { UE-To-Be-Added-ItemIEs} }

UE-To-Be-Added-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-UE-To-Be-Added-Item CRITICALITY reject TYPE UE-To-Be-Added-Item

PRESENCE mandatory },

...

}

-- **************************************************************

--

-- UE ADDITION RESPONSE

--

-- **************************************************************

UEAdditionResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { UEAdditionResponseIEs} },

...

}

UEAdditionResponseIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UE-Added-List CRITICALITY ignore TYPE UE-Added-List PRESENCE optional }|

{ ID id-UE-Failed-To-Add-List CRITICALITY ignore TYPE UE-Release-List PRESENCE optional },

...

}

UE-Added-List ::= SEQUENCE (SIZE(1.. maxInterDUUEs)) OF ProtocolIE-SingleContainer { { UE-Added-ItemIEs} }

UE-Added-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-UE-Added-Item CRITICALITY reject TYPE UE-Added-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

-- UE Deletion ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

-- UE DELETION Request

--

-- **************************************************************

UEDeletionRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container {{ UEDeletionRequestIEs}},

...

}

UEDeletionRequestIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UE-ToBeDeleted-List CRITICALITY reject TYPE UE-List PRESENCE mandatory },

...

}

-- **************************************************************

--

-- UE DELETION RESPONSE

--

-- **************************************************************

UEDeletionResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { UEDeletionResponseIEs} },

...

}

UEDeletionResponseIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory },

...

}

-- **************************************************************

--

-- UE Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE MODIFICATION REQUEST

--

-- **************************************************************

UEModificationRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { UEModificationRequestIEs} },

...

}

UEModificationRequestIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UE-ToBeModified-List CRITICALITY reject TYPE UE-ToBeModified-List PRESENCE mandatory },

...

}

UE-ToBeModified-List ::= SEQUENCE (SIZE(1.. maxInterDUUEs)) OF ProtocolIE-SingleContainer { { UE-ToBeModified-ItemIEs} }

UE-ToBeModified-ItemIEs D2CP-PROTOCOL-IES ::= {

{ID id-UE-ToBeModified-Item CRITICALITY ignore TYPE UE-ToBeModified-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

-- UE MODIFICATION RESPONSE

--

-- **************************************************************

UEModificationResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { UEModificationResponseIEs} },

...

}

UEModificationResponseIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UE-Modified-List CRITICALITY ignore TYPE UE-Modified-List PRESENCE optional }|

{ ID id-UE-FailedToAddMod-List CRITICALITY ignore TYPE UE-Release-List PRESENCE optional },

...

}

UE-Modified-List ::= SEQUENCE (SIZE(1.. maxInterDUUEs)) OF ProtocolIE-SingleContainer { { UE-Modified-ItemIEs} }

UE-Modified-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-UE-Modified-Item CRITICALITY ignore TYPE UE-Modified-Item PRESENCE mandatory},

...

}

-- **************************************************************

--

-- UE MODIFICATION INDICATION

--

-- **************************************************************

UEModificationIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { UEModificationIndicationIEs} },

...

}

UEModificationIndicationIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UE-ToBeUpdated-List CRITICALITY ignore TYPE UE-ToBeUpdated-List PRESENCE optional }|

{ ID id-UE-ToBeReleased-List CRITICALITY ignore TYPE UE-Release-List PRESENCE optional },

...

}

UE-ToBeUpdated-List ::= SEQUENCE (SIZE(1.. maxInterDUUEs)) OF ProtocolIE-SingleContainer { { UE-ToBeUpdated-ItemIEs } }

UE-ToBeUpdated-ItemIEs D2CP-PROTOCOL-IES ::= {

{ ID id-UE-ToBeUpdated-Item CRITICALITY reject TYPE UE-ToBeUpdated-Item PRESENCE mandatory},

...

}

-- **************************************************************

--

-- UE Reconfiguration ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE RECONFIGURATION COMPLETE

--

-- **************************************************************

UEReconfigurationComplete ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { UEReconfigurationCompleteIEs} },

...

}

UEReconfigurationCompleteIEs D2CP-PROTOCOL-IES ::= {

{ ID id-TransactionID CRITICALITY reject TYPE TransactionID PRESENCE mandatory }|

{ ID id-Source-O-DU-ID CRITICALITY reject TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-Target-O-DU-ID CRITICALITY ignore TYPE O-DU-ID PRESENCE mandatory }|

{ ID id-UE-Reconfig-List CRITICALITY reject TYPE UE-Reconfig-List PRESENCE mandatory },

...

}

END

-- ASN1STOP

Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

D2CP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) ngran-access (22) modules (3) d2cp (4) version1 (1) d2cp-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Criticality,

ProcedureCode,

ProtocolIE-ID

FROM D2CP-CommonDataTypes

maxInterDUUEs,

maxInterDUSCells,

maxLCs,

maxSupportedD2U,

maxSupportedPV

FROM D2CP-Constants

ProtocolExtensionContainer{},

D2CP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

D2CP-PROTOCOL-IES

FROM D2CP-Containers;

##### --A

##### --B

BitRate ::= INTEGER (0..4000000000000,...)

##### --C

Cause ::= ENUMERATED {

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

scell-down,

ue-not-allowed,

cell-overload,

cqi-failue,

harq-failure,

pv-notsupported,

udp-invalidinfo,

success,

...

}

CAType		::=	ENUMERATED {dl, ...}

C-RNTI ::= INTEGER (0..65535, ...)

##### --D

D2UEID ::= INTEGER (0..maxInterDUUEs, ...)

DRX-Config ::= OCTET STRING

D2U-Info-Item	::= SEQUENCE {

d2u-udp-info	D2U-UDP-Info-List,

pvList			ProtocolVersion-List,

t-clearTB		T-ClearTB,

iE-Extensions ProtocolExtensionContainer { { D2U-Info-Item-ExtIEs } } OPTIONAL,

...

}

D2U-Info-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

D2U-UDP-Info-List	::= SEQUENCE (SIZE (1.. maxSupportedD2U)) OF D2U-UDP-Info-Item

D2U-UDP-Info-Item	::= SEQUENCE {

d2u-ip	 		TransportLayerAddress,

d2u-port			PortNum,

iE-Extensions ProtocolExtensionContainer { { D2U-UDP-Info-Item-ExtIEs } } OPTIONAL,

...

}

D2U-UDP-Info-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

##### --E

##### --F

##### --G

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink 		BitRate,

maxFlowBitRateUplink 			BitRate,

guaranteedFlowBitRateDownlink BitRate,

guaranteedFlowBitRateUplink 	BitRate,

maxPacketLossRateDownlink 		INTEGER(0..1000) OPTIONAL,

maxPacketLossRateUplink 		INTEGER(0..1000) OPTIONAL,

iE-Extensions ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

##### --H

##### --I

##### --J

##### --K

##### --L

LCID ::= INTEGER (1..32, ...)

LC-Info-List ::= SEQUENCE (SIZE(1..maxLCs)) OF LC-Info-Item

LC-Info-Item ::= SEQUENCE {

lCID 					LCID,

qoSInformation 	QoSInformation,

sNSSAI				SNSSAI,

iE-Extensions 		ProtocolExtensionContainer { { LC-Info-ItemExtIEs } } OPTIONAL,

...

}

LC-Info-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

LC-List ::= SEQUENCE (SIZE(1..maxLCs)) OF LC-Item

LC-Item ::= SEQUENCE {

lCID LCID,

iE-Extensions ProtocolExtensionContainer { { LC-ItemExtIEs } } OPTIONAL,

...

}

LC-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

LC-Release-List	::= SEQUENCE (SIZE(1..maxLCs)) OF LC-Release-Item

LC-Release-Item	::= SEQUENCE {

lCID LCID,

cause Cause,

iE-Extensions ProtocolExtensionContainer { { LC-Release-ItemExtIEs } } OPTIONAL,

...

}

LC-Release-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

##### --M

MaxMIMO-Layers	::=INTEGER (1..16, ...)

MeasGapConfig	::= OCTET STRING

##### --N

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel PriorityLevel,

pre-emptionCapability Pre-emptionCapability,

pre-emptionVulnerability Pre-emptionVulnerability,

iE-Extensions ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

NGRANAllocationAndRetentionPriority-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

NRCellIdentity ::= BIT STRING (SIZE(36))

NRCGI ::= SEQUENCE {

pLMN-Identity PLMN-Identity,

nRCellIdentity NRCellIdentity,

iE-Extensions ProtocolExtensionContainer { {NRCGI-ExtIEs} } OPTIONAL,

...

}

NRCGI-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

NRPCI ::= INTEGER(0..1007)

##### --O

O-DU-ID ::= INTEGER (0..68719476735)

##### --P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar PER-Scalar,

pER-Exponent PER-Exponent,

iE-Extensions ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} } OPTIONAL,

...

}

PacketErrorRate-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PeriodicCqiReportSize ::= INTEGER (1..50, ...)

PLMN-Identity ::= OCTET STRING (SIZE(3))

PortNum ::= BIT STRING (SIZE (16))

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel ::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

ProtocolVersion-List	::= SEQUENCE (SIZE (1.. maxSupportedPV)) OF ProtocolVersion-Item

ProtocolVersion-Item	::= SEQUENCE {

pv		INTEGER(0..255),

iE-Extensions ProtocolExtensionContainer { { ProtocolVersion-Item-ExtIEs } } OPTIONAL,

...

}

ProtocolVersion-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

P-O-DU-To-S-O-DU-SCell-Information ::= SEQUENCE {

sCellID					NRCGI,

sCellIndex				SCellIndex,

servingCellMO			ServingCellMO		OPTIONAL,

maxMIMO-LayersDL		MaxMIMO-Layers		OPTIONAL,

reportConfigType		OCTET STRING,

iE-Extensions ProtocolExtensionContainer { { P-O-DU-To-S-O-DU-SCell-Information-ExtIEs } } OPTIONAL,

...

}

P-O-DU-To-S-O-DU-SCell-Information-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

##### --Q

QoSInformation ::= SEQUENCE {

nr5QI								INTEGER (0..255, ...),

nGRANallocationRetentionPriority	NGRANAllocationAndRetentionPriority,

qoSPriorityLevel					INTEGER (1..127),

packetDelayBudget					PacketDelayBudget,

packetErrorRate					PacketErrorRate,

delayCritical						ENUMERATED {delay-critical, non-delay-critical},

gbrQoSInfo							GBR-QoSFlowInformation				OPTIONAL,

iE-Extensions ProtocolExtensionContainer { { QoSInformation-ExtIEs } } OPTIONAL,

...

}

QoSInformation-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

##### --R

##### Result	::= ENUMERATED {success, failure}--S

S-O-DU-To-P-O-DU-SCell-Information ::= SEQUENCE {

sCellID						NRCGI,

sCellIndex					SCellIndex,

sCellConfig					SCellConfig,

maxMIMO-LayersUsedDL		MaxMIMO-Layers,

periodicCqiReportSize	PeriodicCqiReportSize	OPTIONAL,

csi-ReportConfig			OCTET STRING,

iE-Extensions ProtocolExtensionContainer { { S-O-DU-To-P-O-DU-SCell-Information-ExtIEs } } OPTIONAL,

...

}

S-O-DU-To-P-O-DU-SCell-Information-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

SCell-Added-List  ::= SEQUENCE (SIZE(1..maxInterDUSCells)) OF SCell-Added-Item

SCell-Added-Item ::= SEQUENCE {

sO-DU-to-pO-DU-SCell-Information S-O-DU-To-P-O-DU-SCell-Information,

iE-Extensions ProtocolExtensionContainer { { SCell-Added-ItemExtIEs } } OPTIONAL,

...

}

SCell-Added-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

SCellConfig ::= OCTET STRING

SCellIndex ::=INTEGER (1..31, ...)

SCell-List ::= SEQUENCE (SIZE(1..maxInterDUSCells)) OF SCell-Item

SCell-Item ::= SEQUENCE {

sCellID					NRCGI,

sCellIndex				SCellIndex,

iE-Extensions ProtocolExtensionContainer { { SCell-ItemExtIEs } } OPTIONAL,

...

}

SCell-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

SCell-Info-List ::= SEQUENCE {

pO-DU-To-sO-DU-SCell-Information P-O-DU-To-S-O-DU-SCell-Information,

iE-Extensions ProtocolExtensionContainer { { SCell-Info-ListExtIEs } } OPTIONAL,

...

}

SCell-Info-ListExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

SCell-Release-List ::= SEQUENCE (SIZE(1..maxInterDUSCells)) OF SCell-Release-Item

SCell-Release-Item ::= SEQUENCE {

sCellID					NRCGI,

sCellIndex				SCellIndex,

cause						Cause,

iE-Extensions ProtocolExtensionContainer { { SCell-Release-ItemExtIEs } } OPTIONAL,

...

}

SCell-Release-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

ServingCellMO ::= INTEGER (1..64, ...)

Served-Cell-Item  ::= SEQUENCE{

servedCellInformation 		OCTET STRING,

caType				CAType,

startCRNTI			C-RNTI,

endCRNTI			C-RNTI,

iE-Extensions ProtocolExtensionContainer { { Served-Cell-Item-ExtIEs } } OPTIONAL,

...

}

Served-Cell-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellToMod-Item ::= SEQUENCE{

oldNRCGI			NRCGI,

servedCellInformation 		OCTET STRING,

caType				CAType,

startCRNTI			C-RNTI,

endCRNTI			C-RNTI,

iE-Extensions ProtocolExtensionContainer { { ServedCellToMod-Item-ExtIEs } } OPTIONAL,

...

}

ServedCellToMod-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellToRel-Item ::= SEQUENCE{

nrCGI			NRCGI,

iE-Extensions ProtocolExtensionContainer { { ServedCellToRel-Item-ExtIEs } } OPTIONAL,

...

}

ServedCellToRel-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI ::= SEQUENCE {

sST 	OCTET STRING (SIZE(1)),

sD 		OCTET STRING (SIZE(3)) OPTIONAL ,

iE-Extensions ProtocolExtensionContainer { { SNSSAI-ExtIEs } } OPTIONAL,

...

}

SNSSAI-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

##### --T

T-ClearTB		::= ENUMERATED { ms1, ms2, ms5, ms10, ms20, ms60, ...}

TimeToWait		::= ENUMERATED { s1, s2, s5, s10, s20, s60, s120, ...}

TransactionID ::= INTEGER (0..255, ...)

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))

##### --U

UE-Added-Item ::= SEQUENCE {

cRNTI						C-RNTI,

d2UEID					D2UEID,

sCellAddedList			SCell-Added-List ,

lcAddedList				LC-List,

lcFailedToAddList		LC-Release-List,

sCellFailedToAddList	SCell-Release-List,

iE-Extensions 			ProtocolExtensionContainer { { UE-Added-Item-ExtIEs} } OPTIONAL,

...

}

UE-Added-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::={

...

}

UE-List ::= SEQUENCE (SIZE(1..maxInterDUUEs)) OF UE-Item

UE-Item ::= SEQUENCE {

cRNTI	C-RNTI,

d2UEID	D2UEID,

iE-Extensions ProtocolExtensionContainer { { UE-ItemExtIEs } } OPTIONAL,

...

}

UE-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

UE-Reconfig-List ::= SEQUENCE (SIZE(1..maxInterDUUEs)) OF UE-Reconfig-Item

UE-Reconfig-Item ::= SEQUENCE {

cRNTI	C-RNTI,

d2UEID	D2UEID,

sCellList	SCell-List										,

iE-Extensions ProtocolExtensionContainer { { UE-Reconfig-ItemExtIEs } } OPTIONAL,

...

}

UE-Reconfig-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

UE-Modified-Item ::= SEQUENCE {

cRNTI						C-RNTI,

d2UEID						D2UEID,

sCellUpdatedList			SCell-Added-List			OPTIONAL,

lcUpdatedList				LC-List 					OPTIONAL,

lcFailedToAddModList		LC-Release-List			OPTIONAL,

sCellFailedToAddModList		SCell-Release-List		OPTIONAL,

iE-Extensions ProtocolExtensionContainer { { UE-Modified-Item-ExtIEs} } OPTIONAL,

...

}

UE-Modified-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::={

...

}

UE-Release-List ::= SEQUENCE (SIZE(1..maxInterDUUEs)) OF UE-Release-Item

UE-Release-Item ::= SEQUENCE {

cRNTI	C-RNTI,

d2UEID	D2UEID,

cause	Cause,

iE-Extensions ProtocolExtensionContainer { { UE-Release-ItemExtIEs } } OPTIONAL,

...

}

UE-Release-ItemExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

UE-To-Be-Added-Item ::= SEQUENCE {

cRNTI									C-RNTI,

d2UEID								D2UEID,

maxMIMO-LayersDL-PerDU			INTEGER (1..32, ...),

uE-CapabilityRATContainerList	UE-CapabilityRATContainerList,

sCellToBeAddedList				SCell-Info-List,

measGapConfig						MeasGapConfig	OPTIONAL,

drx-Config							DRX-Config		OPTIONAL,

lcToBeAddedList					LC-Info-List,

iE-Extensions ProtocolExtensionContainer { { UE-To-Be-Added-Item-ExtIEs} } OPTIONAL,

...

}

UE-To-Be-Added-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

UE-ToBeModified-Item	::= SEQUENCE {

cRNTI							C-RNTI,

d2UEID							D2UEID,

maxMIMO-LayersDL-PerDU			MaxMIMO-Layers,

uE-CapabilityRATContainerList	UE-CapabilityRATContainerList	OPTIONAL,

sCellToAddModList				SCell-Info-List		OPTIONAL,

measGapConfig					MeasGapConfig		OPTIONAL,

drx-Config						DRX-Config			OPTIONAL,

lcToAddModList					LC-Info-List		OPTIONAL,

lcToBeDeletedList				LC-List				OPTIONAL,

sCellToBeDeletedList			SCell-List			OPTIONAL,

iE-Extensions ProtocolExtensionContainer { { UE-ToBeModified-Item-ExtIEs} } OPTIONAL,

...

}

UE-ToBeModified-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

UE-ToBeUpdated-Item ::= SEQUENCE {

cRNTI							C-RNTI,

d2UEID							D2UEID,

sCellToBeReleasedList			SCell-Release-List				OPTIONAL,

lcToBeReleasedList				LC-Release-List					OPTIONAL,

iE-Extensions ProtocolExtensionContainer { { UE-ToBeUpdated-Item-ExtIEs} } OPTIONAL,

...

}

UE-ToBeUpdated-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

UE-CapabilityRATContainerList ::= OCTET STRING

UpdatedCell-Item ::= SEQUENCE{

nRCGI				NRCGI,

nrPCI				NRPCI	OPTIONAL,

result				Result,

iE-Extensions ProtocolExtensionContainer { { UpdatedCell-Item-ExtIEs} } OPTIONAL,

...

}

UpdatedCell-Item-ExtIEs D2CP-PROTOCOL-EXTENSION ::= {

...

}

##### --V

##### --W

##### --X

##### --Y

##### --Z

END

-- ASN1STOP

Common Definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

D2CP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) d2cp (4) version1 (1) d2cp-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

-- Common Data Types

--

-- **************************************************************

Criticality ::= ENUMERATED { reject, ignore, notify }

Presence ::= ENUMERATED { optional, conditional, mandatory }

ProcedureCode ::= INTEGER (0..255)

ProtocolExtensionID ::= INTEGER (0..maxProtocolExtensions)

ProtocolIE-ID ::= INTEGER (0..maxProtocolIEs)

END

-- ASN1STOP

Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

D2CP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) d2cp (4) version1 (1) d2cp-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM D2CP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-d2Setup								ProcedureCode ::= 0

id-d2errorIndication 				ProcedureCode ::= 1

id-d2ConfigurationUpdate			ProcedureCode ::= 2

id-d2Release							ProcedureCode ::= 3

id-uEAddition							ProcedureCode ::= 4

id-uEDeletion							ProcedureCode ::= 5

id-uEModificationUpdate				ProcedureCode ::= 6

id-uEModificationIndication		ProcedureCode ::= 7

id-uEReconfigurationComplete		ProcedureCode ::= 8

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxInterDUUEs			INTEGER ::= 65535

maxInterDUSCells		INTEGER ::= 32

maxLCs					INTEGER ::= 255

maxSupportedD2U		INTEGER ::= 255

maxSupportedPV			INTEGER ::= 16

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause									ProtocolIE-ID ::= 0

id-D2U-Info-List						ProtocolIE-ID ::= 1

id-D2U-Info-Item						ProtocolIE-ID ::= 2

id-Source-O-DU-ID						ProtocolIE-ID ::= 3

id-Target-O-DU-ID						ProtocolIE-ID ::= 4

id-TransactionID						ProtocolIE-ID ::= 5

id-Served-Cell-Item					ProtocolIE-ID ::= 6

id-Served-Cell-List					ProtocolIE-ID ::= 7

id-ServedCellToAdd-Item				ProtocolIE-ID ::= 8

id-ServedCellToAdd-List				ProtocolIE-ID ::= 9

id-ServedCellToMod-Item 			ProtocolIE-ID ::= 10

id-ServedCellToMod-List				ProtocolIE-ID ::= 11

id-ServedCellToRel-Item				ProtocolIE-ID ::= 12

id-ServedCellToRel-List				ProtocolIE-ID ::= 13

id-UE-Added-Item						ProtocolIE-ID ::= 14

id-UE-Added-List						ProtocolIE-ID ::= 15

id-UE-Failed-To-Add-Item			ProtocolIE-ID ::= 16

id-UE-Failed-To-Add-List			ProtocolIE-ID ::= 17

id-UE-FailedToAddMod-List 			ProtocolIE-ID ::= 18

id-UE-Item								ProtocolIE-ID ::= 19

id-UE-List								ProtocolIE-ID ::= 20

id-UE-Modified-Item 					ProtocolIE-ID ::= 21

id-UE-Modified-List					ProtocolIE-ID ::= 22

id-UE-To-Be-Added-Item				ProtocolIE-ID ::= 23

id-UE-To-Be-Added-List				ProtocolIE-ID ::= 24

id-UE-ToBeDeleted-List				ProtocolIE-ID ::= 25

id-UE-ToBeModified-Item				ProtocolIE-ID ::= 26

id-UE-ToBeModified-List				ProtocolIE-ID ::= 27

id-UE-ToBeUpdated-Item 				ProtocolIE-ID ::= 28

id-UE-ToBeUpdated-List				ProtocolIE-ID ::= 29

id-UE-ToBeReleased-List				ProtocolIE-ID ::= 30

id-UE-Reconfig-List					ProtocolIE-ID ::= 31

id-UpdatedCell-Item 					ProtocolIE-ID ::= 32

id-UpdatedCell-List					ProtocolIE-ID ::= 33

id-TimeToWait							ProtocolIE-ID ::= 34

END

-- ASN1STOP

Container Definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

D2CP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) d2cp (4) version1 (1) d2cp-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

ProtocolIE-ID,

ProtocolExtensionID

FROM D2CP-CommonDataTypes;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

D2CP-PROTOCOL-IES ::= CLASS {

&id 				ProtocolIE-ID 		UNIQUE,

&criticality 	Criticality,

&Value,

&presence 		Presence

}

WITH SYNTAX {

ID 					&id

CRITICALITY 		&criticality

TYPE 					&Value

PRESENCE				&presence

}

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

D2CP-PROTOCOL-IES-PAIR ::= CLASS {

&id 							ProtocolIE-ID 			UNIQUE,

&firstCriticality 		Criticality,

&FirstValue,

&secondCriticality 		Criticality,

&SecondValue,

&presence 					Presence

}

WITH SYNTAX {

ID 							&id

FIRST CRITICALITY 		&firstCriticality

FIRST TYPE 					&FirstValue

SECOND CRITICALITY 		&secondCriticality

SECOND TYPE 				&SecondValue

PRESENCE 					&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

D2CP-PROTOCOL-EXTENSION ::= CLASS {

&id 					ProtocolExtensionID 				UNIQUE,

&criticality 			Criticality,

&Extension,

&presence 				Presence

}

WITH SYNTAX {

ID 						&id

CRITICALITY 			&criticality

EXTENSION				&Extension

PRESENCE 				&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {D2CP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer {D2CP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {D2CP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id 				D2CP-PROTOCOL-IES.&id 					({IEsSetParam}),

criticality 	D2CP-PROTOCOL-IES.&criticality 		({IEsSetParam}{@id}),

value 			D2CP-PROTOCOL-IES.&Value 				({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {D2CP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {D2CP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id 						D2CP-PROTOCOL-IES-PAIR.&id 						({IEsSetParam}),

firstCriticality		D2CP-PROTOCOL-IES-PAIR.&firstCriticality 		({IEsSetParam}{@id}),

firstValue 				D2CP-PROTOCOL-IES-PAIR.&FirstValue 				({IEsSetParam}{@id}),

secondCriticality 	D2CP-PROTOCOL-IES-PAIR.&secondCriticality 	({IEsSetParam}{@id}),

secondValue 			D2CP-PROTOCOL-IES-PAIR.&SecondValue 			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, D2CP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, D2CP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {D2CP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {D2CP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id 						D2CP-PROTOCOL-EXTENSION.&id 				({ExtensionSetParam}),

criticality 			D2CP-PROTOCOL-EXTENSION.&criticality 	({ExtensionSetParam}{@id}),

extensionValue 		D2CP-PROTOCOL-EXTENSION.&Extension 		({ExtensionSetParam}{@id})

}

-- **************************************************************

END

-- ASN1STOP

Message Transfer Syntax

D2CP shall use the ASN.1 Basic Packed Encoding Rules (BASIC-PER) Aligned Variant as transfer syntax, as specified in ITU-T Rec. X.691 [12].

Timers

D2 User Plane protocol

D2UP services

The following services are provided by the D2UP protocol:

- Provision of D2 sequence number information for user data request and transfer between SCell O-DU and PCell O-DU.

- Request of new user data or retransmission scheduling on SCell O-DU along with the slot Range where UE can be scheduled in SCell O-DU.

- Request for data transfer along with information on allocated DL channel (PDSCH & PDCCH) resources on the SCell(s)

- Provision of sharing PUCCH resource allocation with SCell O-DU.

- Information of clearing the MAC TB stored in SCell O-DU once data is transmitted to UE successfully.

- Information of clearing the MAC TB once PCell O-DU decides not to continue with further transmission

- sharing of the Uplink Control Information received from UE.

D2UP Functions

The functions of D2UP are described in TS O-RAN.WG5.TS.D2GAP [2].

D2UP Procedures

Downlink data transfer

DL Scheduling Request

The purpose of the DL SCHEDULING REQUEST message as shown in Figure 6.3.1.1-1, is to request sO-DU to schedule the UE for transmission of the new data or retransmission of the data that is already transmitted on SCell. This message may also be used for releasing the earlier used HARQ process, if pO-DU decides to abort the retransmission, or to extend the slot range for UE scheduling in sO-DU. pO-DU sends asynchronous requests to the sO-DU including the cumulated data size in the Buffer Length IE, that may be scheduled by the sO-DU.

pO-DU may share information for multiple UEs and multiple SCells in a single DL SCHEDULING REQUEST message.

If the pO-DU triggers DL SCHEDULING REQUEST message for a D2 UE ID before reception of DL DATA REQUIRED message for the same D2 UE ID, it shall set D2 Sched SN IE to 0. Otherwise D2 Sched SN shall be set to the value of D2 Sched SN received in the latest DL DATA REQUIRED message for the UE from sO-DU.

PCell O-DU

SCell O-DU

DL SCHEDULING REQUEST

Figure 6.3.1.1-1: DL scheduling request

sO-DU may use D2 UE data size IE to determine the size of information for each UE, identified by D2 UE ID IE, that is received in DL SCHEDULING REQUEST message.

If the DT IE indicates “reTx”:

- If HARQCount IE is not set to zero, HARQStatus IE for each HARQ PID indicates the cause of HARQ failure. sO-DU shall initiate the retransmission, subject to the allowed range of slots as specified by the SlotOffset and SlotRange IE, on all the HARQ processes identified by HARQ PID for all the SCells that are included in the message and identified by SCellIndex IE as defined in 3GPP TS 38.331 clause 6.3.2 [5]. If clearTB IE is set to TRUE, sO-DU shall delete the stored MAC TB and reuse the HARQ process for further new data transmission.

- If HARQCount IE is received as zero, sO-DU shall consider the received message to increase the range of slots, as specified in SlotRange IE, for the retransmission.

If the DT IE indicates “newTx”:

- If LCCount IE is not set to 0, the sO-DU shall determine the SCells using SCell Bitmap IE, where data transmission may be scheduled for the UE. Buffer length IE shall be used to determine the size of data for each LCID that needs to be scheduled in SCells. The sO-DU, if it can schedule the UE within the slots as mentioned in SlotRange/SlotMask, shall allocate the HARQ PID and request for PUCCH resources from pO-DU by triggering DL DATA REQUIRED message.

- If LCCount IE is received as 0, sO-DU shall interpret the received message to increase the range of slots, as specified in SlotRange/SlotMask IE, for UE scheduling of new transmissions.

If the DT IE indicates “Both”:

- DL SCHEDULING REQUEST message shall contain IEs applicable for both reTx and newTx.

- All the conditions listed above for reTx and newTx shall be applicable.

In case sO-DU is unable to schedule the UE in allowed slot range, sO-DU shall discard all the DL SCHEDULING REQUEST that were received for this slot range.

DL Data Required

DL DATA REQUIRED message, as shown in Figure 6.3.1.2-1, is used by sO-DU to request the MAC TB and PUCCH resources from pO-DU in case of successful scheduling of the UEs on SCell.

This message may encode information about multiple D2 UE ID. pO-DU may use the D2 UE data size to determine the size of the received message for each D2 UEID.

sO-DU shall manage the sequence number per UE. The sequence number shall be incremented by 1 each time a DL DATA REQUIRED message for a UE is sent by the same sO-DU. D2 Sched SN IE shall be set to the value of sequence number.

PCell O-DU

SCell O-DU

DL DATA REQUIRED

If DT IE is set to “reTx”, sO-DU shall set the LCCount to 0 and LCID and Buffer length IE shall not be included in the message. sO-DU shall not set the DT IE as “Both”.

Figure 6.3.1.2-1: DL data required

pO-DU shall use the SCellIndex IE to identify each SCell where UE is scheduled for transmission with MAC TB of size indicated in TBSize IE. OTA Slot/ SFN, HARQ PID and PDCCH start symbol IEs for each of SCell shall be used by pO-DU to allocate the PUCCH resources. If pO-DU encounters any kind of failure (PUCCH resource allocation failure or any other failure), it shall send PUCCH RESOURCE TRANSFER message with result IE set to Failure. DL DATA TRANSFER message shall not be sent in this case.

If MACCEPresent IE is set to TRUE, pO-DU shall include all the MAC CEs indicated by MACCECount while forming the MAC TB. MAC CE Length and MAC CE Data IEs indicate the length of MAC CE and MAC CE data. pO-DU shall use “FV” IE to identify the MAC CE type, whether fixed length or variable length. MAC TB encoding will be done according to 3GPP TS 38.321[20].

If DT IE is set to “newTx”:

- pO-DU shall use the Buffer Length IE to determine the size of data for each LC, identified by LCID, that is scheduled for transmission.

- HARQ PID shall be set as the one allocated by the sO-DU for transmission of the data.

If DT IE is set to “reTx”:

- LCID and Buffer length IE shall be considered as not present in the message and next octet shall be read as Extension octet if present.

- HARQ PID shall be set as the one that was used during initial transmission and received from pO-DU in DL SCHEDULING REQUEST.

DL Data Transfer

As shown in Figure 6.3.1.3-1, DL DATA TRANSFER message is used by pO-DU to transfer the MAC TB to sO-DU for transmission to UE on the specified OTA slot / OTA SFN corresponding to SCell where data is scheduled in sO-DU. This message contains MAC TB for a UE scheduled on a SCell. If pO-DU encounters any kind of failure (PUCCH resource allocation failure or any other failure), it shall not send DL DATA TRANSFER message. pO-DU may segment the MAC TB into multiple segments depending on the MAC TB size.

PCell O-DU

SCell O-DU

DL DATA TRANSFER

Figure 6.3.1.3-1: DL data transfer

sO-DU may use the D2 UE data size to determine the size of the received message for the UE identified by D2 UE ID, for which segments are received. sO-DU uses D2 Sched SN and OTA Slot/OTA SFN for mapping the DL DATA REQUIRED and DL DATA TRANSFER message on a SCell identified by SCellIndex.

sO-DU shall determine the number of segments belonging to a single MAC TB using Total Segment count IE. sO-DU shall concatenate the data mentioned in TBSegment IE for each segment to form a MAC TB using Segment SN, Segment Offset and Segment Length IE.

sO-DU stores the complete MAC TB for retransmission purposes. sO-DU shall delete the MAC TB and reuse the HARQ PID for new transmission in the following conditions:

- Reception of successful transmission indication (HARQ feedback) from pO-DU

- Reception of ClearTB IE set to TRUE from pO-DU.

- If none of the above conditions are met and for new transmission in the following conditions

  - T_TBClear timer expires.

PUCCH Resource Transfer

PUCCH RESOURCE TRANSFER message, as shown in Figure 6.3.1.4-1, is used by pO-DU to share the allocated PUCCH resources with sO-DU.

PCell O-DU

SCell O-DU

PUCCH RESOURCE TRANSFER

Figure 6.3.1.4-1: PUCCH resource transfer

If information for multiple UEs is encoded in the message as indicated by D2 UE Count IE, sO-DU uses the D2 UE data size to determine the size of information for each UE identified by D2 UE ID for which information is received in PUCCH RESOURCE TRANSFER message.

D2 Sched SN, OTA Slot/ OTA SFN is used by sO-DU to map the allocated PUCCH resources for the SCell(s) identified by SCellIndex as defined in 3GPP TS 38.331[5] clause 6.3.2, with the corresponding DL DATA REQUIRED message.

UCI Transfer

As shown in Figure 6.3.2-1, UCI Transfer procedure is used by pO-DU to send the received UCI information from UE to sO-DU. pO-DU shares CSI information or HARQ feedback using UCI TRANSFER message.

If CSI data for multiple SCells are multiplexed, pO-DU shall parse the CSI information intended for the SCells configured in the sO-DU for the UE as defined in 3GPP TS 38.212[18] clause 6.3.1.1.2. The raw data per cell shall be sent in CSI Data IE to sO-DU.

PCell O-DU

SCell O-DU

UCI TRANSFER

Figure 6.3.2-1: UCI transfer

If information for multiple UEs is encoded in the message as indicated by D2 UE Count IE, sO-DU may use D2 UE data size to determine the size of information for each of UE identified by D2 UE ID. SCell data size IE may be used to parse the information for each SCell identified by SCellIndex IE.

If CSIStatus is set to “Not Present”, “Failure”, or “DTX”:

- sO-DU shall consider that CSI data is not present in the UCI TRANSFER message and value of CSI Data Length shall be considered as 0.

- Octets containing CSI Data IE shall not be present in the message.

If CSIStatus is set to “Present”:

- sO-DU reads the CSI Data Length and interprets the CSI Data to determine the reported CQI, RI, L1-RSRP, PMI etc as defined in 3GPP TS 38.213[17] clause 9.2 and 9.3.

If HARQCount is encoded as 0, sO-DU shall interpret that HARQ feedback is not present in the message. Otherwise HARQCount specifies the number of HARQ processes for which HARQ feedback is present in the received message. For each HARQ process identified by HARQ PID IE,

If HARQStatus is set to “Ack”:

- sO-DU shall interpret it as successful reception of data by UE.

- sO-DU shall clean up the HARQ process and may use it for further new data transmission for the same UE. ClearTB IE is ignored for this scenario.

If HARQStatus is set to “DTX” or “Nack”:

- sO-DU shall handle the received negative HARQ feedback.

- If ClearTB IE is set to TRUE, sO-DU deletes the TB and shall use the HARQ PID for the new data transmission with toggled NDI bit.

- If ClearTB is set to FALSE, sO-DU shall continue storing data and start the T_ClearTB timer. sO-DU waits for DL SCHEDULING REQUEST for triggering the retransmission. On reception of DL SCHEDULING REQUEST T_ClearTB timer shall be stopped.

If DL SCHEDULING REQUEST indicating retransmission request is not received until expiry of T_ClearTB timer, sO-DU deletes the TB and shall use the HARQ PID for the new data transmission with toggled NDI bit.

Elements of D2UP Communication

    - General

In the present document the structure of D2UP frames is specified by using figures similar to Figure 6.4.1-1.

| Bits |  |  |  |  |  |  |  | Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| Field 1 |  |  |  | Field 2 |  |  |  | 1 |
| Field 3 |  |  |  |  |  | Field 4 |  |  |
| Field 4 continues |  |  |  | spare |  |  |  |  |
| spare |  |  |  | Field 5 |  |  |  |  |
| Field 6 |  |  |  |  |  |  |  |  |
| Field 6 continues |  |  |  | Padding bits |  |  |  |  |
| Future extension |  |  |  |  |  |  |  |  |
| padding |  |  |  |  |  |  |  |  |

Figure 6.4.1-1: Example frame format for D2UP

Unless otherwise indicated, fields which consist of multiple bits within an octet have the most significant bit located at

the highest bit position of the field (according to the bit numbers indicated above frame in Figure 6.4.1-1). In addition, if

a field spans several octets, most significant bits are located in lower numbered octets (right of frame in Figure 6.4.1-1).

The frame is transmitted starting from the lowest numbered octet. Within each octet, the bits are sent according to

decreasing bit position (bit position 7 first).

Bits labelled "Spare or Reserved" should be set to "0" by the sender and should not be checked by the receiver.

The header part of the frame is always an integer number of octets. The payload part is octet aligned (by adding

'Padding Bits' when needed). The total size of the frame shall not exceed 1018 octets (see TS 29.281 [19]).

The receiver should be able to remove an additional Future Extension field that may be present.

See description of Future Extension field in A.1.

Message Format for the D2UP

All D2-u messages begin with a common header as shown in Figure 6.4.2-1.

| Bits |  |  |  |  |  |  |  | Number of octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PV |  |  |  | Application ID |  |  |  | 1 |
| Message Type |  |  |  |  |  |  |  | 1 |

Figure 6.4.2-1: D2-u header

DL SCHEDULING REQUEST

This message format is used by pO-DU to sO-DU to request DL scheduling for a UE(s). It also conveys the information of allowed slots where UE can be scheduled in SCell(s).

This message also can be used to increase the range of slots where UE can be scheduled in SCell

This message is used to request both new data scheduling and for re-transmission scheduling on sO-DU.

SCellCount and SCellIndex IEs in octet 8 are relevant only for retransmission scenarios. These fields shall not be considered for new transmission.

IEs listed in octets 13-24 or 28 are related to new transmission of the data. If DT is set to “newTx” or “Both”, Octets 13 - 23 shall be present in the message and convey information associated for new data transmission requests. These octets are not present otherwise.

IEs listed in octets 8-12 except DT IE, are related to re-transmission of the data. If DT is set to “reTx” or “Both”,  octets 8-12 shall be present in the message and convey information associated for HARQ status and retransmission requests. These octets are not present, otherwise.

If SlotType IE is set to 1, bits 0-3 in the same octet and the next 4 octets shall be considered for indicating the SlotMask. If SlotType is set to 0, bits 0-3 in the octet shall be reserved and next octet (1 octet) shall be used to determine the range of slots.

SlotOffset/SlotRange/SlotMask IEs received in latest DL SCHEDULING REQUEST shall be applied immediately. Slot configuration received in earlier DL SCHEDULING REQUEST shall be overwritten with the newly received one.

Figure 6.4.2.1-1 shows the message format for DL SCHEDULING REQUEST message

| Bits |  |  |  |  |  |  |  | Number of octets | Octet No |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| D2 UE Count |  |  |  |  |  |  |  | 1 | 1 |
| D2 UE ID |  |  |  |  |  |  |  | 2 | 2 |
| D2 Sched SN |  |  |  |  |  |  |  | 2 | 4 |
| D2 UE data size |  |  |  |  |  |  |  | 2 | 6 |
| DT |  | SCellCount |  |  |  | E1 |  | 1 | 8 |
| SCellIndex |  |  |  |  |  |  |  | 1 | 9 |
| SlotOffset |  |  | SlotRange |  |  |  |  | 1 | 10 |
| HARQCount |  |  |  | R | R | R | R | 1 | 11 |
| HARQ PID |  |  |  | HARQStatus |  | ClearTB |  | 1 | 12 |
| SlotType | SlotOffset |  |  | Reserved/SlotMask |  |  |  | 1 | 13 |
| SlotRange/SlotMask |  |  |  |  |  |  |  | 1 or 4 | 14 or 14-17 |
| SCell Bitmap |  |  |  |  |  |  |  | 4 | 15-18 or 18-21 |
| LCCount |  |  |  | E2 |  | R | R | 1 | 19 or 22 |
| LCID |  |  |  |  |  |  |  | 1 | 20 or 23 |
| Buffer Length |  |  |  |  |  |  |  | 3 | 21-24 or 24-26 |
| Extension Octet1 |  |  |  |  |  |  |  | 1 | 24 or 27 |
| Extension Octet2 |  |  |  |  |  |  |  | 1 | 25 or 28 |

Figure 6.4.2.1-1: DL SCHEDULING REQUEST message format

DL DATA REQUIRED

sO-DU uses this message format to request MAC TB and/or PUCCH resources for the transmission on a SCell in response to a DL SCHEDULING REQUEST message.

Figure 6.4.2.2-1 shows the DL DATA REQUIRED message

| Bits |  |  |  |  |  |  |  | Number of octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| D2 UE Count |  |  |  |  |  |  |  | 1 |
| D2 UE ID |  |  |  |  |  |  |  | 2 |
| D2 Sched SN |  |  |  |  |  |  |  | 2 |
| TotalPendingBO |  |  |  |  |  |  |  | 4 |
| D2 UE data size |  |  |  |  |  |  |  | 2 |
| SCellCount |  |  |  | E1 |  | R | R | 1 |
| SCellIndex |  |  |  |  |  |  |  | 1 |
| TBSize |  |  |  |  |  |  |  | 3 |
| MACCEPresent | MACCECount |  |  |  | R | R | R | 1 |
| FV | MACCELength |  |  |  |  |  |  | 1 |
| MAC CE Data |  |  |  |  |  |  |  | MACCElength |
| OTA SFN |  |  |  |  |  |  |  | 1 |
| OTA SFN |  | OTA Slot |  |  |  |  |  | 1 |
| OTA Slot |  | HARQ PID |  |  |  | PDCCH Start Symbol |  | 1 |
| PDCCH Start Symbol |  | Curr MCS |  |  |  |  | Curr RI | 1 |
| Curr RI |  | DT |  | LCCount |  |  |  | 1 |
| LCID |  |  |  |  |  |  |  | 1 |
| Buffer Length |  |  |  |  |  |  |  | 3 |
| Extension Octet |  |  |  |  |  |  |  | 1 |

Figure 6.4.2.2-1: DL DATA REQUIRED message format

DL DATA TRANSFER

Message format shown in Figure 6.4.2.3-1 is used by pO-DU to share the MAC TB as requested by sO-DU in DL DATA REQUIRED message.  This format is used for new data transmission.

| Bits |  |  |  |  |  |  |  | Number of octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| D2 UE data size |  |  |  |  |  |  |  | 2 |
| D2 UE ID |  |  |  |  |  |  |  | 2 |
| D2 Sched SN |  |  |  |  |  |  |  | 2 |
| SCellIndex |  |  |  |  |  |  |  | 1 |
| OTA SFN |  |  |  |  |  |  |  | 1 |
| OTA SFN |  | OTA Slot |  |  |  |  |  | 1 |
| OTA Slot |  | E1 |  | R | R | R | R | 1 |
| TBSize |  |  |  |  |  |  |  | 3 |
| Total Segment count |  |  |  |  |  |  |  | 1 |
| Segment SN |  |  |  |  |  |  |  | 1 |
| Segment Offset |  |  |  |  |  |  |  | 3 |
| Segment Length |  |  |  |  |  |  |  | 2 |
| TBSegment |  |  |  |  |  |  |  | n |
| Extension Octet |  |  |  |  |  |  |  | 1 |

Figure 6.4.2.3-1: DL DATA TRANSFER message format

PUCCH RESOURCE TRANSFER

Message format in Figure 6.4.2.4-1 is used by pO-DU to transfer the allocated PUCCH resources to sO-DU in response of DL DATA REQUIRED message. sO-DU shall use the information while encoding the DCI message to send to the UE for scheduling DL data.

If PUCCH allocation in pO-DU is not successful in response to a DL DATA REQUIRED message for a D2 UE ID, the result field shall be set as “Failure” and subsequent fields may be filled with 0 by pO-DU. sO-DU shall ignore these fields.

| Bits |  |  |  |  |  |  |  | Number of octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| D2 UE Count |  |  |  |  |  |  |  | 1 |
| D2 UE ID |  |  |  |  |  |  |  | 2 |
| D2 Sched SN |  |  |  |  |  |  |  | 2 |
| D2 UE data size |  |  |  |  |  |  |  | 2 |
| OTA SFN |  |  |  |  |  |  |  | 1 |
| OTA SFN |  | OTA Slot |  |  |  |  |  | 1 |
| OTA Slot |  | SCellCount |  |  |  | E1 |  | 1 |
| SCellIndex |  |  |  |  |  |  |  | 1 |
| Result |  | cDAI |  |  | K1 |  |  | 1 |
| PUCCH Rsrc Ind |  |  |  |  | tDAI |  |  | 1 |
| Extension Octet |  |  |  |  |  |  |  | 1 |

Figure 6.4.2.4-1: PUCCH RESOURCE TRANSFER message format

UCI TRANSFER

UCI Transfer message format as shown in Figure 6.4.2.5-1 is used by pO-DU to send the received CSI or HARQ feedback information from UE which are intended for sO-DU.

This message format is also used to clear the HARQ process and stored TB in sO-DU if PCell aborts the retransmission in sO-DU.

| Bits |  |  |  |  |  |  |  | Number of octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| D2 UE Count |  |  |  |  |  |  |  | 1 |
| D2 UE ID |  |  |  |  |  |  |  | 2 |
| D2 UE data size |  |  |  |  |  |  |  | 2 |
| ScellCount |  |  |  | E1 |  | R | R | 1 |
| ScellIndex |  |  |  |  |  |  |  | 1 |
| SCell data size |  |  |  |  |  |  |  | 1 |
| CSIStatus |  | CSI Data Length |  |  |  |  |  | 1 |
| CSI Data |  |  |  |  |  |  |  | CSI Data Length |
| HARQCount |  |  |  | R | R | R | R | 1 |
| HARQ PID |  |  |  | HARQStatus |  | ClearTB |  | 1 |
| HARQ PID |  |  |  | HARQStatus |  | ClearTB |  | 1 |
| Extension Octet |  |  |  |  |  |  |  | 1 |

Figure 6.4.2.5-1: UCI TRANSFER message format

Coding of information elements in frames

PV: Protocol version

Description: Protocol version indicates the D2-u protocol version. In the present document, version 1 is used.

Value range: 1-15. 0 is reserved.

Field length: 4 bits

Application ID

Description: Indicates the application that is using the D2 interface. In the present document, D2 interface is used for supporting Inter O-DU CA.

Value range: 1-15. 0 is reserved. 1 is defined for Inter O-DU CA.

Field length: 4 bits

Message Type

Description: Indicates the D2-U message IDs as mentioned in .

Value Range: 0-255

Field Value: Table 6.4.3.3-1 below defines all the message type defined for D2-u.

Table 6.4.3.3-1: Message ID

| Message ID | Name |
| --- | --- |
| 1 | DL SCHEDULING REQUEST. |
| 2 | DL DATA REQUIRED |
| 3 | DL DATA TRANSFER |
| 4 | PUCCH RESOURCE TRANSFER |
| 5 | UCI TRANSFER |
| 0 & 6-255 | Reserved |

D2 UE Count

Description: Indicates the number of D2 UE IDs for which information is encoded in a message.

Value range: 1-255.

Field length: 8 bits

D2 UE ID

Description: Unique number to identify the UE in the pO-DU and sO-DU. pO-DU assigns D2 UE ID to a UE using UE ADDITION REQUEST message and it does not change until UE is released.

Value range: 0-65535

Field length: 16 bits

D2 Sched SN

Description: Sequence number per UE which is generated by sO-DU for each DL DATA REQUIRED message.  pO-DU returns the same value in each of the DL DATA TRANSFER and PUCCH RESOURCE TRANSFER messages associated with DL DATA REQUIRED message. DL SCHEDULING REQUEST message encodes the D2 Sched SN as the one last received in DL DATA REQUIRED message.

Value range: 1-65535. 0 is used in DL SCHEDULING REQUEST message when it is sent before receiving any DL DATA REQUIRED message.

Field length: 16 bits

D2 UE data size

Description: Indicates the size of information in octets or bytes for each UE received that is encoded in D2 message .

Value range: 1-9000

Field length: 16 bits

SCellCount

Description: Count of SCells for which information is encoded in the message

Value range: 1-15

Field length: 4 bits

SCellIndex

Description: Index of SCell as defined in 3GPP TS 38.331 clause 6.3.2[5].

Value range: 1-31

Field length: 8 bits

SCellBitmap

Description: Each bit indicates the SCellIndex of the cell where UE is allowed to be scheduled for inter O-DU CA in sO-DU. Bit position indicates the SCellIndex as defined in 3GPP TS 38.331 clause 6.3.2[5]. For example bit position 0 is for PCell. Bit Position 1 indicates SCell with SCellIndex 1 and so on. Value at the bit position indicates whether a UE is allowed to be scheduled on the SCell or not.

Bit value 1: UE is allowed to be scheduled on the SCell with SCellIndex as bit position.

Bit value 0: UE is not allowed to be scheduled on the SCell with SCellIndex as bit position.

Value range: 0x00000002 – 0xfffffffe.

Field length: 32 bits

SlotType

Description: Indicates if SlotRange or SlotMask is encoded for UE scheduling in sO-DU. If SlotType indicates presence of SlotRange, the least significant four bits in the octet carrying SlotType are reserved.

0: indicates that SlotRange is configured . SlotRange is defined in section 6.4.3.13.

1: indicates that SlotMask is configured for UE scheduling. SlotMask is 36 bit field as defined in section 6.4.3.14

Value range: 0-1

Field length: 1 bit

SlotOffset

Description: Indicates the slot offset for UE scheduling with respect to receiver slot. sO-DU shall schedule UE after slot offset only. If value received is 0, it means, UE shall be considered for scheduling from next slot onwards. This is set as per the highest sub carrier spacing among all the SCells configured for the UE in sO-DU.

Value range: 0-7

Field length: 3 bits

SlotRange

Description: Continuous range of slots in SCell starting from SlotOffset where UE can be scheduled by the sO-DU. This is configurable parameter. This is set as per the highest sub carrier spacing among all the SCells configured for the UE in sO-DU.

Value range: 1-50 ( for new transmission) and 1- 31(for retransmission)

Field length: 5 bits for retransmission and 8 bits for new transmission.

SlotMask

Description: Indicates the slots starting from SlotOffset where UE can be scheduled in SCell. Either Slot Range or Slot Mask will be present which is indicated by Slot Type. Each bit indicates the slot where UE can be scheduled by sO-DU. This is set as per the highest sub carrier spacing among all the SCells configured for the UE in sO-DU.

EXAMPLE 1: SlotOffset = 0, SlotMask = 0x01indicates UE can be scheduled in SCell only in next slot in which the message is received.

EXAMPLE 2: SlotOffset = 1, SlotMask = 0x01 indicates that UE can only be scheduled in SCell in slot number 2 if message is received in slot #0.

EXAMPLE 3: SlotOffset = 1, SlotRange = 10 indicates that UE can only be scheduled in SCell from slot number 2 until slot number 12, if message is received in slot #0.

Value range: 1-36

Field length: 36 bits

TotalPendingBO

Description: Indicates the total pending BO at the sO-DU per UE basis. This field contains the value of total BO pending at the sO-DU while sending the DL DATA REQUIRED message.  Total pending BO will be calculated in sO-DU based on the BO received in latest DL SCHEDULING REQUEST. Size of scheduled data will be subtracted from this BO while calculating the TotalPendingBO. Primary O-DU may use this information to determine the pending data size in sO-DU.  0xFFFFFFFF indicates that field TotalPendingBO information is not encoded by sO-DU hence does not contain a valid value to be used.

Value range: 0- 0xFFFFFFFF

Field length: 32 bits

TBSize

Description: Indicates the total MAC TB size scheduled for transmission by SCell in a sO-DU. pO-DU shall form the TB of size indicated by TBSize and send it to sO-DU. If MAC CE Data is present, it shall be included while forming the MAC TB for transmission.

Value range: 1- 600000 bytes. All other values are reserved.

Field length: 24 bits

MACCEPresent

Description: Flag indicates whether MAC CE is present in the message or not.

MACCEPresent  = 0 : MAC CE field is not present.

MACCEPresent  = 1: MAC CE field is present.

Value range: 0-1

Field length: 1 bit

MACCECount

Description: This field indicates the number of MAC CEs that are present in the message. These MAC CEs are used by pO-DU while forming the TB as per the TBSize sent by sO-DU. value 0 means, MAC CE is not present in the message.

Value range: 0-15

Field length: 4 bits

FV

Description: Flag indicating whether MAC CE is of fixed length or variable length.

NOTE: pO-DU uses this information when forming the MAC TB in accordance with 3GPP MAC TS 38.321[20]

FV = 0 :  Fixed length MAC CE.

FV = 1 :  Variable length MAC CE

Value range: 0-1

Field length: 1 bit

MACCELength

Description: Indicates the length of MAC CE field in bytes.

Value range: 1-127

Field length: 7 bits

MACCEData

Description: Data containing the MAC CE of size MACCELength.

Field length: As per MACCELength

OTA SFN

Description: Sub frame number where PDCCH transmission is scheduled for UE by sO-DU with respect to a given SCell.

Value range: 0 -  1023

Field length: 10 bit

OTA Slot

Description: Slot number where PDCCH transmission is scheduled for UE by sO-DU with respect to a given SCell.

Value range: 0-160. All other values are reserved.

Field length: 8 bits

PDCCH Start Symbol

Description: Start symbol of PDCCH where DCI is scheduled for a UE by a SCell of a sO-DU. This information is used by pO-DU while allocating the PUCCH resources for SCells.

Value range: 0-13. 14 and 15 is reserved value.

Field length: 4 bits

CurrRI

Description: Rank Indicator used in the SCell of the sO-DU for the UE where UE is scheduled.

Value range: 0-7. 0 indicates that RI is 8.

Field length: 3 bits

CurrMCS

Description: MCS used in the SCell of the sO-DU for the UE where UE is scheduled.

Value range: 0-28

Field length: 5 bits

DT: Data Type

Description: Indicates Whether message contains information for new data transmission or for retransmission or for both.

DT= 0 : newTx

DT=1: reTx

DT=2:  Both. It means message contains information for both new transmission as well as for retransmission. This value cannot be encoded in DL DATA REQUIRED message.

Value range: 0-2. 3 is reserved in present document.

Field length: 2 bits

LCCount

Description: This field indicates the number of logical channels (LCs) for which information is encoded in the message.

Value range: 0-15.

Field length: 4 bits

LCID

Description: Indicates the logical channel identity for which information is encoded in the message. This is same ID as defined in 3GPP TS 38.321[20] clause 6.2.1

Value range: as specified in 3GPP TS 38.321 [20] clause 6.2.1.

Field length: 8 bits

Buffer Length

Description: Indicates the length of bufferin octets for each LCID that can be scheduled on SCell(s).

Value range: 1- 1200000

Field length: 24 bits

Total Segment Count

Description: Indicates the total number of D2 segments that are formed out of MAC TB as specified by TBSize in DL DATA REQUIRED message.

Value range: 1-255

Field length: 8 bits

Segment SN

Description: Sequence number of D2 message that contains the segments of a MAC TB if it was segmented by pO-DU.

Value range: 1- Total Segment Count

Field length: 8 bits

Segment Offset

Description: Indicates the offset in octets of received D2 segment in the MAC TB to form the complete TB.

Value range: 1- 600000

Field length: 24 bits

Segment Length

Description: it is the number of octets in a segment encoded in TBSegment field.

Value range: 1- 9000

Field length: 16 bits

TBSegment

Description:  Field containing MAC TB segment that is scheduled for transmission on SCell.

Field length: Segment Length bytes

Result

Description: Indicates the status of PUCCH allocation by pO-DU. On reception of DL DATA REQUIRED messages, if pO-DU is unable to allocate PUCCH resources or due to any other failure, unable to send data to sO-DU, result flag can be set to false. sO-DU will not transmit to the UE in corresponding SCell on the OTA SFN/Slot.

Result = 1: PUCCH allocation is successful

Result = 0 – Failure at pO-DU hence no data transmission to the UE in corresponding SCell on the OTA SFN/Slot. Other values are reserved in the present document.

Value range: 0-1

Field length: 2 bits

cDAI: counter Downlink Assignment Index

Description: Indicates the counter downlink assignment index as defined in 3GPP TS 38.213[17] clause 9.1.3 that need to be encoded in DCI message.

Value range: 0- 4

Field length: 3 bits

tDAI: Total Downlink Assignment Index

Description: Indicates the  total downlink assignment index as defined in 3GPP TS 38.213[17] clause 9.1.3  on the OTA slot/SFN. This information is sent to UE in DCI message.

Value range: 0- 4

Field length: 3 bits

K1

Description: Indicates the slot difference between PDSCH reception and PUCCH transmission by the UE as defined in 3GPP TS 38.213[17] clause 9.2.3.

Value range: 1-8

Field length: 3 bits

PUCCH Rsrc Ind

Description:  PUCCH Resource indication as defined in 3GPP TS 38.213[17] clause 9.2.3.

Field length: 5 bits

SCell data size

Description:  indicates the size of control data in octet or bytes for each SCell.

Field length: 8 bits

HARQCount

Description: This field indicates the number of HARQ processes for which information is encoded in the message.

Value range: 0- 15.

Field length: 4 bits

HARQ PID

Description: Indicates the HARQ PID as defined in 3GPP TS 38.212 [18] clause 7.3.1.2, for which information is encoded in the message. . HARQ PID is not released until MAC TB is successfully received by the UE or retransmission is aborted.

Value range: 0-15

Field length: 4 bits

HARQStatus

Description: Indicates the HARQ feedback status. It can be ACK, NACK, DTX depending on received feedback from UE or if nothing is received on the scheduled slot.

HARQStatus = 0 (ACK)

HARQStatus = 1 (NACK)

HARQStatus = 2 (DTX)

Value range: 0-2

Field length: 2 bits

ClearTB

Description: Flag indicates to sO-DU if stored TB for HARQ PID has to be cleared. pO-DU sets the value to 1 on reception of HARQ ACK or in case of aborting the retransmission on the SCell.

ClearTB =1 :  Delete the MAC TB stored in sO-DU and clear the HARQ PID.

ClearTB =0 :  MAC TB continued to be stored in sO-DU.

Value range: 0-1.

Field length: 2 bits

CSIStatus

Description: Indicates the status of CSI data in the D2 message.

CSIStatus = 0 :  Success, CSI data is present.

CSIStatus = 1  :  CRC failure or CSI decoding failure. In this case CSI Length is set to 0 and CSI data is not present.

CSIStatus = 2  : DTX at the OTA Slot/SFN where CSI data was scheduled. In this case CSI Length is set to 0 and CSI data is not present.

Value range: 0-2

Field length: 2 bits

CSI Data Length

Description:  Indicates the length of CSI data in octets or bytes in the D2 message for specified Scell of a D2 UE ID.

Value range: 0- 63 bytes.

Field length: 6 bits

CSI Data

Description:  The raw CSI data for the SCell as received from UE containing information such as CQI, CRI, RI, PMI information as defined in 3GPP TS 38.213[17] clause 9.2.5.

Field length: CSI Data Length bytes

E1

Description:  This bit indicates the presence of extension octet in all the messages.

E1 =0: Extension octet is not present in the message.

E1 = 1 : Extension Octet ( In DL SCHEDULING REQUEST, Extension Octet 1) field per UE per SCell is present in the message

E1 =2 : Extension Octet ( In DL SCHEDULING REQUEST, Extension Octet 1) field per UE is present in the message

Field length: 2bits

E2

Description:  This field indicates the presence of extension octet2 in D2 messages. This is per UE information.

E2 =0: Extension octet2 is not present in the message.

E2 =1: Extension Octet2 is present.

Field length: 2 bits

Extension Octet/ Extension Octet1/ Extension Octet2

Description:  This is an octet which can be used in future if any extension is required in the message. Its present is decided by E1/E2 field.

Field length: 8 bits

- Annex A (informative):
Examples for DL SCHEDULING REQUEST

- A.1: 	New Data transmission:

Figure A.1-1 mentions the fields that shall be present if DT is set to newTx. Following are the asssumptions in this example:

- In this example, information for 2 UEs are encoded with D2 UE ID as 1 and 2.

- First scheduling request for D2 UE ID 1 is sent before reception of DL DATA REQUIRED from sO-DU. For D2 UE ID 2, it is assumed that DL DATA REQUIRED with D2 Sched SN =1 is received.

- There is single SCell present for the D2 UE ID 1 in sO-DU and SCellIndex is 1 for the SCell.

- There are 2 SCells configured in sO-DU for D2 UE ID 2 with SCellIndex 1 and 2 for each of the SCell.

- SlotType is set to SlotRange for both the UEs.

- Each UE has 1 logical channel configured with LCID =4.

| Bits |  |  |  |  |  |  |  | No Of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| D2 UE Count =2 |  |  |  |  |  |  |  | 1 |
| D2 UE ID =1 |  |  |  |  |  |  |  | 2 |
| D2 Sched SN = 0 |  |  |  |  |  |  |  | 2 |
| D2 UE data size =12 |  |  |  |  |  |  |  | 2 |
| DT = 0 |  | SCellCount = 0 |  |  |  | E1 =0 |  | 1 |
| SlotType = 0 | SlotOffset =2 |  |  | Reserved |  |  |  | 1 |
| SlotRange |  |  |  |  |  |  |  | 1 |
| SCell Bitmap = 0x00000001 |  |  |  |  |  |  |  | 4 |
| LCCount =1 |  |  |  | E2 =0 |  | R | R | 1 |
| LCID = 4 |  |  |  |  |  |  |  | 1 |
| Buffer Length = x |  |  |  |  |  |  |  | 3 |
| D2 UE ID =2 |  |  |  |  |  |  |  | 2 |
| D2 Sched SN = 1 |  |  |  |  |  |  |  | 2 |
| D2 UE data size =12 |  |  |  |  |  |  |  | 2 |
| DT = 0 |  | SCellCount =0 |  |  |  | E1 =0 |  | 1 |
| SlotType = 0 | SlotOffset =2 |  |  | Reserved |  |  |  | 1 |
| SlotRange |  |  |  |  |  |  |  | 1 |
| SCell Bitmap = 0x00000011 |  |  |  |  |  |  |  | 4 |
| LCCount =1 |  |  |  | E2 =0 |  | R | R | 1 |
| LCID = 4 |  |  |  |  |  |  |  | 1 |
| Buffer Length =y |  |  |  |  |  |  |  | 3 |

Figure A.1-1: DL SCHEDULING REQUEST with newTx with SlotType as SlotRange

NOTE: Buffer Length x and y in the above table indicates the size of data that can be scheduled in SCells as mentioned in SCell Bitmap for the UE configured in sO-DU.

Following are the assumptions in Figure A.1-2 for newTx:

- In this example, information for 2 UEs are encoded with D2 UE ID as 1 and 2.

- Scheduling request for D2 UE ID 1 is sent before reception of DL DATA REQUIRED from sO-DU. For D2 UE ID 2, it is assumed that DL DATA REQUIRED with D2 Sched SN =1 is received.

- There is single SCell present for the D2 UE ID 1 in sO-DU and SCellIndex is 1 for the SCell.

- There are 2 SCells configured in sO-DU for D2 UE ID 2 with SCellIndex 1 and 2 for each of the SCell.

- SlotType is set to SlotMask for both the UEs.

- Each UE has 1 logical channel configured with LCID =4.

| Bits |  |  |  |  |  |  |  | No Of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| D2 UE Count =2 |  |  |  |  |  |  |  | 1 |
| D2 UE ID =1 |  |  |  |  |  |  |  | 2 |
| D2 Sched SN = 0 |  |  |  |  |  |  |  | 2 |
| D2 UE data size =15 |  |  |  |  |  |  |  | 2 |
| DT = 0 |  | SCellCount = 0 |  |  |  | E1 =0 |  | 1 |
| SlotType = 1 | SlotOffset =2 |  |  | SlotMask |  |  |  | 1 |
| SlotMask |  |  |  |  |  |  |  | 4 |
| SCell Bitmap = 0x00000001 |  |  |  |  |  |  |  | 4 |
| LCCount =1 |  |  |  | E2 =0 |  | R | R | 1 |
| LCID =4 |  |  |  |  |  |  |  | 1 |
| Buffer Length =x |  |  |  |  |  |  |  | 3 |
| D2 UE ID =2 |  |  |  |  |  |  |  | 2 |
| D2 Sched SN = 1 |  |  |  |  |  |  |  | 2 |
| D2 UE data size = 15 |  |  |  |  |  |  |  | 2 |
| DT = 0 |  | SCellCount =0 |  |  |  | E1 =0 |  | 1 |
| SlotType = 0 | SlotOffset =2 |  |  | SlotMask |  |  |  | 1 |
| SlotMask |  |  |  |  |  |  |  | 4 |
| SCell Bitmap = 0x00000003 |  |  |  |  |  |  |  | 4 |
| LCCount =1 |  |  |  | E2 =0 |  | R | R | 1 |
| LCID =4 |  |  |  |  |  |  |  | 1 |
| Buffer Length =y |  |  |  |  |  |  |  | 3 |

Figure A.1-2: DL SCHEDULING REQUEST with newTx with SlotType as SlotMask

NOTE: Buffer Length x and y in the above table indicates the size of data that can be scheduled in SCells as mentioned in SCell Bitmap for the UE configured in sO-DU.

- A.2:	Retransmission

Figure A.2-1 indicates the fields that shall be present if DT is set to reTx . Following are the assumptions in this example:

- Information for 2 UEs are encoded with D2 UE ID as 1 and 2.

- There is single SCell present for both the UEs with SCellIndex = 1.

- Each UE has status of 2 HARQ processes with PID 1 and 2.

- For UE 1, status is Nack for both the HARQ PIDs. For PID 1, ClearTB is set while for other it is not set.

- For UE 2, status for HARQ PID 1 is Ack  while for HARQ PID 2 it is Nack with ClearTB not set.

| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  | No Of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| D2 UE Count =2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| D2 UE ID=1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| D2 Sched SN |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| D2 UE data size = 6 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| DT = 1 (reTx) |  |  | SCellCount = 1 |  |  |  |  |  |  |  | E1 =0 |  |  | 1 |
| SCellIndex = 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| SlotOffset |  |  |  |  | SlotRange |  |  |  |  |  |  |  |  | 1 |
| HARQCount =2 |  |  |  |  |  |  | R |  | R |  | R |  | R | 1 |
| HARQ PID =1 |  |  |  |  |  |  | HARQStatus = 1 |  |  |  | ClearTB =1 |  |  | 1 |
| HARQ PID =2 |  |  |  |  |  |  | HARQStatus = 1 |  |  |  | ClearTB =0 |  |  | 1 |
| D2 UE ID=2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| D2 Sched SN |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| D2 UE data size = 6 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| DT = 1 (reTx) |  |  | SCellCount = 1 |  |  |  |  |  |  |  | E1 =0 |  |  |  | 1 |
| SCellIndex = 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| SlotOffset |  |  |  |  | SlotRange |  |  |  |  |  |  |  |  |  | 1 |
| HARQCount = 2 |  |  |  |  |  |  | R |  | R |  | R |  | R |  | 1 |
| HARQ PID =1 |  |  |  |  |  |  | HARQStatus = 0 |  |  |  | ClearTB = 1 |  |  |  | 1 |
| HARQ PID =2 |  |  |  |  |  |  | HARQStatus = 1 |  |  |  | ClearTB = 0 |  |  |  | 1 |

Figure A.2-1: DL SCHEDULING REQUEST for retransmission

- A.3:	New Data transmission and retransmission (Both)

Figure A.3-1 indicates the fields that shall be present if DT is set to Both. Following are the assumptions in this example:

- Information for 2 UEs are encoded with D2 UE ID as 1 and 2.

- For D2 UE ID 1 scheduling request is sent before reception of DL DATA REQUIRED from sO-DU. For D2 UE ID 2, it is assumed that DL DATA REQUIRED with D2 Sched SN =1 is received.

- There are 2 SCells with SCellIndex 1 and 2 configured in sO-DU for both the UEs.

- There are one Logical channel encoded for each UE with LCID = 4.

- For D2 UE ID 1, SCellIndex 1 is containing the retransmission information while SCellIdnex 2 is containing the information for new transmission.

- For D2 UE ID 2, both the SCells has retransmission data while SCellIndex 1 is having new transmission data information as well.

- SlotType is set to SlotRange for both the D2 UE ID 1 and SlotType is set to SlotMask for D2 UE ID 2.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| D2 UE Count = 2 |  |  |  |  |  |  |  | 1 |
| D2 UE ID =1 |  |  |  |  |  |  |  | 2 |
| D2 Sched SN |  |  |  |  |  |  |  | 2 |
| D2 UE data size = 16 |  |  |  |  |  |  |  | 2 |
| DT = 2 (Both) |  | SCellCount = 1 |  |  |  | E1 =0 |  | 1 |
| SCellIndex =1 |  |  |  |  |  |  |  | 1 |
| SlotOffset |  |  | SlotRange |  |  |  |  | 1 |
| HARQCount = 1 |  |  |  | R | R | R | R | 1 |
| HARQ PID =1 |  |  |  | HARQStatus = 1 |  | ClearTB =0 |  | 1 |
| SlotType = 0 | SlotOffset |  |  | Reserved |  |  |  | 1 |
| SlotRange |  |  |  |  |  |  |  | 1 |
| SCell Bitmap = 0x00000002 |  |  |  |  |  |  |  | 4 |
| LCCount = 1 |  |  |  | E2 |  | R | R | 1 |
| LCID = 4 |  |  |  |  |  |  |  | 1 |
| Buffer Length = x |  |  |  |  |  |  |  | 3 |
| D2 UE ID =2 |  |  |  |  |  |  |  | 2 |
| D2 Sched SN |  |  |  |  |  |  |  | 2 |
| D2 UE data size = 23 |  |  |  |  |  |  |  | 2 |
| DT = 2 (Both) |  | SCellCount = 2 |  |  |  | E1 =0 |  | 1 |
| SCellIndex =1 |  |  |  |  |  |  |  | 1 |
| SlotOffset |  |  | SlotRange |  |  |  |  | 1 |
| HARQCount = 1 |  |  |  | R | R | R | R | 1 |
| HARQ PID = 1 |  |  |  | HARQStatus = 1 |  | ClearTB =0 |  | 1 |
| SCellIndex =2 |  |  |  |  |  |  |  | 1 |
| SlotOffset |  |  | SlotRange |  |  |  |  | 1 |
| HARQCount = |  |  |  | R | R | R | R | 1 |
| HARQ PID = 1 |  |  |  | HARQStatus = 1 |  | ClearTB = 0 |  | 1 |
| SlotType = 1 | SlotOffset |  |  | SlotMask |  |  |  | 1 |
| SlotMask |  |  |  |  |  |  |  | 4 |
| SCell Bitmap = 0x00000001 |  |  |  |  |  |  |  | 4 |
| LCCount = 1 |  |  |  | E2 = 0 |  | R | R | 1 |
| LCID = 1 |  |  |  |  |  |  |  | 1 |
| Buffer Length = y |  |  |  |  |  |  |  | 3 |

Figure A.3-1: DL SCHEDULING REQUEST for transmission and retransmission (Both)

NOTE: Buffer Length x and y in the above table indicates the size of data that can be scheduled in SCells as mentioned in SCell Bitmap for the UE configured in sO-DU.

- Annex (informative): 
Change history/Change request (history)

| Date | Revision | Description |
| --- | --- | --- |
| 23-10-2025 | 00.00.001 | CRs MAV-0015, MAV-0016, MAV-0017 and MAV-0022 are merged. |
| 30-10-2025 | 00.00.002 | CRs NEC-0110, NEC-0111, NEC-0112, MAV-0026 and MAV-0025 are merged. |
| 30-10-2025 | 00.00.003 | Incorporated review comments provided during F2F meeting |
| 06.11.2025 | 00.00.004 | Incorporated review comments shared by NEC and Ericsson |
| 13.11.2025 | 00.00.005 | Incorporated review comments shared by KDDI and Ericsson |
| 17.11.2025 | 00.00.006 | Examples in Annex A is added for DL SCHEDULING REQUEST |
| 25.11.2025 | 00.00.007 | Incorporates all the comments. |